Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14BT3009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED PROCESS EQUIPMENT DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5x 20 = 100 Marks)

(*Extra graph sheet is permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a | Explain in details about the different Engineering materials used for process equipment construction and give its merits and demerits. | CO1 | 15 |
|  | b | Define unit operation. Enumerate and mention their symbol diagram at least 5 equipment. | CO1 | 05 |
| (OR) |  |  |  |  |
| 2. | a | Discuss about the design procedure for 1-1 shell and tube heat exchanger with neat sketch. | CO2 | 10 |
|  | b | Cold fluid is flowing through the heat exchanger at a rate of 15 m3/hr .It enters the heat exchanger at 303K and leaves at 328K.The hot thermic fluid enters the heat exchanger at the rate of 21m3/hr at a temperature of 388K .Find out the area of heat transfer required assuming the flow is counter current and overall heat transfer coefficient be 2490 W/m2 k. Data : Density of cold fluid= 1000 kg /m3, density of thermic fluid = 950 kg/m3,Specific heat of cold fluid= 4.187 KJ/Kg.K, Specific heat of thermic fluid=2.93 KJ/Kg.K. Draw a neat block diagram. | CO2 | 10 |
| 3 | a | Discuss about the design procedure of storage tanks. | CO1 | 10 |
|  | b | A pressure vessel of 3 m3 volume is to be designed for the maximum operating pressure 500 KN/ m2 .The vessel is made up of steel allowable design stress 118 x103 KN/ m2 at working condition. As on elliptic and the top is connected with dished head and design suitable pressure vessel. Data: Joint efficiency=85%, allowing factor=1.1, Knuckle radius =60- 70% of diameter, Crown radius = 850mm and thickness of gasket= 20 mm. | CO2 | 10 |
|  |  | (OR) |  |  |
| 4 | a | Write down the design procedure of Fractional distillation column Step by step with neat sketch. | CO3 | 10 |
|  | b | A mixture of 35mole % A and 65Mole % B is to be separated in a fractionating column. The concentration of A in the distillate is 93Mole % and 65% of all product A is in the distillate. The feed is half vapor and the reflux ratio is to be 4.0.The relative volatility of A to B is 2.0.Calculate the number of theoretical plates in the column and locate the feed plate. | CO3 | 10 |
| 5 |  | Calculate the amount of steam required for concentrating the solution of caustic soda from 28% by weight of solids to 40% by weight of solids in a single effect evaporator. The feed rate in 25000 kg / hr and its temperature is 60 ºC. .The absolute pressure in the evaporator is 0.2 kg/ cm2.Saturated steam at 1.4 kg/ cm2 is to be used as heating medium. The elevation in boiling points is 25 ºC. If the overall heat transfer coefficient is 670 kcal/ hr m2 ºC. Calculate the heating surface required and steam economy. The enthalpy data for various streams are as follows. Vapour at 0.2 kg/ cm2 = 623 kcal/ kg. 28% NaOH at 60 ºC = 50 kcal / kg, 40% NaOH at 85 ºC =90 kcal / kg. Latent heat of steam at 1.4 kg/ cm2 =534 kcal / kg. | CO3 | 20 |
|  |  | (OR) |  |  |
| 6 | a | Write down the design procedure of single effect evaporator with neat sketch. | CO3 | 10 |
|  | b | Discuss about air batch reactor with neat sketch, give them an important design equations. | CO3 | 10 |
| 7 |  | A single effect evaporator is fed with 5000 kg / hr of solution containing 10% solute by weight .The feed temperature is 303K and is to be concentrated to a solution of 20% solute by weight. The evaporator is at atmospheric pressure ( 101.325 Kpa )and area of evaporator is 69m2 .Saturated steam is supplied at 143.3 kpa as a heating medium. Calculate steam economy and overall heat transfer coefficient. Data: Enthalpy of feed at 303K= 125.79 KJ/kg, Enthalpy of vapors at101Kpa= 2676.1 KJ/kg, Enthalpy of product = 419.04 KJ/kg, Enthalpy of saturated steam at 143Kpa= 2691.5KJ/kg, Saturation temperature of steam =110 ºC,Boiling point solution=boiling point of water=373K Enthalpy of saturated water at 383K = 461.30 KJ/Kg. | CO3 | 20 |
|  |  | (OR) |  |  |
| 8 |  | Describe the design of absorption column, NTU, HTU and HETP and differentiate between the regular and random packing. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9 |  | Describe the design of rotary drum dryer with neat diagram and give its industrial applications | CO3 | 20 |