Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May– 2017

| Sub. Code: | 14CE2002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF SOLIDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A C.I. Flat, 300 mm long and of 30 mm x 50 mm uniform section, is acted upon by the following forces uniformly distributed over the respective cross-section; 25 kN in the direction of length(tensile); 350 kN in the direction of width (compressive); and 200 kN in the direction of thickness (tensile). Determine the change in volume of the flat. Take E = 140 GN/m2 and m = 4 | 1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | A steel bolt and sleeve assembly is shown in Fig. The nut is tightened up on the tube through the rigid end blocks until the tensile force in the bolt in 40 kN. If an external load 30 kN is then applied to the end blocks, tending to pull them apart, estimate the resulting force in the bolt and sleeve. | 1 | 20 |
| 3. | a. | The principle stresses at a point in a body are 100 MN/m2 (tensile) and 60 MN/m2 (compressive). Determine the normal stress and the shear stress on a plane inclined at 50˚ to the axis of the major principal stress. | 1 | 5 |
|  | b. | A body is subjected to tensile stresses of 30N/mm2 and 10N/mm2 acting on planes at right angles to each other. They are accompanied by a shear stress of magnitude 10 N/mm2. Using Mohr’s circle, determine the magnitudes and directions of principal stresses and also greatest shear stress. | 1 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | At a point in the strained material is subjected to stresses as shown in figure. Locate the principal planes and evaluate the principal stresses. | 1 | 15 |
|  | b. | Define principal planes and principal stresses and explain their uses. | 1 | 5 |
| 5. |  | For the beam shown in figure, calculate the value of U.D.L. wkN/m so that ending moment at C is 50 kNm. Draw the S.F and B.M diagrams for this beam and locate the point of contraflexure. B D A C 2 m 4 m 4 m w kN/m 20 kN | 2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Draw the Shear force and Bending moment diagrams for the beam shown in the figure indicating principal values. | 2 | 20 |
| 7. |  | An I section shown in figure is simply supported over a span of 12 m. If maximum permissible bending stress is 80 N/mm2, what concentrated load can be carried at a distance of 4 m from one support? | 2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Determine the diameter of a solid shaft which will transmit 300 kW at 250 r.p.m. The maximum shear stress should not exceed 30N/mm2 and the twist should not be more than 1 degree in a shaft length of 2m. Take modulus of rigidity = 1 x 105 N/mm2. | 3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | A beam AB of span 8m is simply supported at the ends. It carries an uniformly distributed load of 30 kN/m over its entire length and a concentrated load of 60 kN at 3m from the support A. Determine the maximum deflection in the beam and the location where the deflection occurs. Take Young’s modulus = 200 GN/m2, Moment of inertia = 80 x 10-4 m4 | 3 | 20 |

ALL THE BEST