Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14CS3008 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ANALYSIS ARCHITECTURE AND DESIGN OF NETWORKS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Show how performance boundaries and thresholds could be used in the following scenarios. - An application has a service requirement for round-trip delay to be less than 100 ms. If delay is greater than 100 ms, notify the network administrator. - A user requires capacity of up to 512 Kb/s but may not exceed 1.5 Mb/s. You want to keep track of how much time the user's capacity is between 512 Kb/s and 1.5 Mb/s. | CO2 | 10 |
|  | b. | Figure shows the uptime (U) and down time (D) of the network for 6 weeks. Assume the total uptime and down time is equal to 1800 hours and 408 hours respectively. Calculate Availability of the network. | CO2 | 5 |
|  | c. | Differentiate hierarchy and diversity characteristics of networks. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch the template of Requirement specification Map. Prepare a requirement analysis for a company LAN. | CO1 | 10 |
|  | b. | Summarize about the various user requirements used for setting up the network architecture. | CO1 | 10 |
| 3. | a. | Differentiate mission-critical, rate-critical, real-time and non-real time applications. Give examples for each type of applications. | CO1 | 10 |
|  | b. | Discuss the necessity of developing RMA requirements in high performance networks. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe the suitable flow model for each set of devices and applications listed here. Label each as either a unidirectional or a bidirectional flow. - Client-server application: downstream : 1.2 Mb/s capacity, upstream: 15 Kb/s capacity. - Streaming video from video server to a subscriber's PC: 300 Kb/s capacity, 40-ms delay (one way). - Downloading pages from the Web: downstream, 250 Kb/s capacity, 5-second delay; upstream, 100 Kb/s capacity. - Transaction processing from point-of-sale (PoS) machine to server: upstream: 30 Kb/s capacity, 100-ms round-trip delay, downstream: 50 Kb/s capacity. | CO2 | 10 |
|  | b. | A networking company gets the contract of deploying a nework for an educational institution which has 4 LANs each with its own server. The company wanted to build a flow model to interconnect these LANs. Analyze the following flow models and propose a suitable flow models for the company. - Peer-to-peer - Client-server - Hierarchical client-server - Distributed computing | CO1 | 10 |
| 5. | a. | Given network address is 192.168.124.0/24, develop a variable-length addressing scheme that best fits the design, with the following numbers of users: AS Number; Location; Department; Users. 1; Chicago Campus Building 1; Legal; 120. Accounting; 13. Chicago Campus Building 2; HQ; 16. Engineering; 30. 2; Toronto; Sales; 5. Boston; Sales; 5. 3; Philadelphia; Operations1; 30. Operations2; 5. Sales; 14. | CO2 | 10 |
|  | b. | Discuss the significance of the following network functions over Architectural models such as Topological, Flow based and Functional models. - Addressing and Routing - Network management - Performance - Security | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Refer to the exhibit. Apply Link State Routing (LSR)algorithm to find a best path from the source node ‘D’ to destination node ‘C’. | CO2 | 10 |
|  | b. | Tabulate the dependencies between each of the following performance mechanisms: QoS, SLA and Policies | CO1 | 10 |
| 7. | a. | For each of the SNMP commands get, get-next, and set, describe what each command does and give an example of how it is used. | CO2 | 10 |
|  | b. | Elucidate the most common performance mechanisms of internet architecture. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Apply the security mechanisms to support the following requirements. Show where each mechanism might be applied for the following. - An intranet between each of the routers connected to the WAN. - Remote access security must be provided for each of the 15 dial-up routers connected to the LAN in Washington, DC. - All traffic flows between Los Angeles and Minneapolis must be encrypted. | CO2 | 10 |
|  | b. | Develop a sample threat analysis for a network with which you are familiar. Show assets and potential threats that must be identified. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | List the differences between the network blueprints, network diagrams, and component plans. Why would a network design have sets of each of these? | CO1 | 10 |
|  | b. | Which of the following evaluation criteria most likely apply to equipment evaluations, which ones apply to service-provider evaluations, and which apply to both? - Available service-level agreements (SLAs) - Standards compliance (IETF) - Mean time between failure (MTBF) - Mean time between service outage (MTBSO) - Hardware scalability | CO2 | 10 |

ALL THE BEST