Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Sub. Code: | 14CS3062 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ROUTING AND SWITCHING TECHNOLOGIES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Summarize the importance of ip addresses, ARP protocol, ICMP protocol. | CO1 | 10 |
|  | b. | For the following host addresses and subnet masks, find what subnet each address belongs to, the broadcast address of that subnet, and the range of host addresses for that subnet: 192.168.27.70/26 172.25.0.235/27 10.7.16.37/19 | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | The static route configurations for the routers in figure are as follows: RTA ip route 172.20.96.0 255.255.240.0 172.20.20.1 ip route 172.20.82.0 255.255.240.0 172.20.20.1 ip route 172.20.64.0 255.255.240.0 172.20.20.1 ip route 172.20.160.0 255.255.240.0 172.20.30.255 ip route 172.20.144.0 255.255.240.0 172.20.30.255 ip route 172.20.128.0 255.255.240.0 172.20.30.255 RTB ip route 172.20.192.0 255.255.240.0 172.20.16.50 ip route 172.20.224.0 255.255.240.0 172.20.16.50 ip route 172.20.128.0 255.255.240.0 172.20.16.50 ip route 172.20.160.0 255.255.240.0 172.20.30.255 ip route 172.20.144.0 255.255.240.0 172.20.30.255 ip route 172.20.128.0 255.255.240.0 172.20.30.255 RTC ip route 172.20.192.0 255.255.240.0 172.20.16.50 ip route 172.20.208.0 255.255.255.0 172.20.16.50 ip route 172.20.224.0 255.255.240.0 172.20.16.50 ip route 172.20.96.0 255.255.240.0 172.20.20.1 ip route 172.20.82.0 255.255.240.0 172.20.20.1 ip route 172.20.64.0 255.255.240.0 172.20.20.1 Users are complaining of several connectivity problems in this internet. Find the mistakes in the static route configurations. | CO1 | 12 |
|  | b. | You have been told to configure 192.168.13.175 on an interface with a mask of 255.255.255.240. Is there a problem? If so, what is it? | CO1 | 3 |
|  | c. | What is the counting-to-infinity problem, and how can it be controlled? | CO1 | 2 |
|  | d. | What is a routing protocol? | CO1 | 2 |
|  | e. | What is the purpose of route invalidation timers? | CO1 | 1 |
| 3. | a. | Compare and contrast between static routing and dynamic routing protocols. | CO1 | 10 |
|  | b. | Discuss how unequal-cost load balancing is accomplished in routers running IGRP routing protocol. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | What is the multicast address used by RIPv2? What is the advantage of multicasting messages over broadcasting them? | CO2 | 4 |
|  | b. | A router is configured to redistribute between EIGRP and IGRP, as follows: router eigrp 15 network 192.168.5.0 no auto-summary metric weights 0 1 1 0 1 1 ! router igrp 5 network 172.16.0.0 metric weights 0 0 0 1 1 1 The routers in the EIGRP domain are not learning routes into the IGRP domain, and routers in the IGRP domain are not learning routes into the EIGRP domain. What is wrong? Explain. | CO2 | 16 |
| 5. |  | Explain the operations of OSPF routing protocol. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the implementation of search algorithms in bridge address table. | CO2 | 20 |
| 7. | a. | In practical scenario, redundant links are created to avoid complete network failure in an event of failure of one link, however it creates loops in the network path. Explain the loop elimination mechanism implemented at LAN bridges. | CO2 | 10 |
|  | b. | Discuss any one aspects of source routing operation. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the issues in link aggregation. | CO2 | 10 |
|  | b. | Illustrate the mechanism used by switches to improve performance both of the network and of the switches if the ports connected to the switches are operating in half duplex mode. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain VLAN concepts. | CO3 | 20 |

ALL THE BEST