Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14EC2065 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | INFORMATION THEORY AND CODING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Find an expression for average information content of symbols in long independent sequences. | CO1 | 5 |
|  | b. | For the second order Markov source as shown in figure with binary source alphabet (s=0,1), find: i) state probabilities ii) entropy of each state iii) entropy of source iv) Show that G1>G2>H(s). | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | A non symmetric binary channel shown in figure has a symbol rate of 1000 symbols/sec. i) Find H(X), H(Y), H(X,Y), H(Y/X), I(XY). Take P(X=0)=, P(X=1)=, =0.75, =0.9. ii) Find the capacity of the binary symmetric channel when ==0.75. | CO1 | 15 |
|  | b. | State the properities of mutual information. | CO1 | 5 |
| 3. | a. | Explain the error detection and correction capabilities of Hamming codes. | CO3 | 8 |
|  | b. | Apply Huffman coding procedure to following message ensemble and determine average length of encoded message also. Determine the coding efficiency. Use coding alphabet D=4, there are 10 symbols. X=[x1, x2, x3,……….,x10] P[X]=[0.17, 0.16, 0.15, 0.1, 0.08, 0.05, 0.05, 0.04, 0.1, 0.1] | CO3 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | Use differential entropy to compare the randomness of random variables. | CO2 | 4 |
|  | b. | A four symbol alphabet has following probabilities Pr(a0) =1/2 Pr(a0) =1/4 Pr(a0) =1/8 Pr(a0) =1/8 and an entropy of 1.75 bits. Determine a codebook for this four letter alphabet that satisfies source coding theorem. | CO2 | 12 |
|  | c. | Derive the entropy for a binary symmetric channel. | CO2 | 4 |
| 5. | a. | Distinguish between block codes and convolutional codes. | CO3 | 4 |
|  | b. | Write syndrome properities of linear block codes. | CO3 | 4 |
|  | c. | Consider (7,4) linear block code defined by the generator matrix, G= . Given that is sent and signal pattern is received. Calculate syndrome and decoding pattern. | CO3 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | a) Draw the encoder for a rate ½, constraint length k=4. The impulse response of path 1 and path 2 of the encoder is g1=(1,0,1,0) and g2=(1,1,0,1) respectively. b) Find the coded output produced by message sequence 10010 using transform domain approach. | CO3 | 15 |
|  | b. | Draw the diagram of encoder and syndrome calculator generated by polynomial g(x). | CO3 | 5 |
| 7. | a. | Construct the syndrome evaluation table for the (7,4) cyclic code by generator polynomial g(x) =1+x+x3. Calculate the code word for the message sequence 1001 and Construct systematic generator matrix G. | CO3 | 16 |
|  | b. | Justify why cyclic codes are extremely well suited for error detection? | CO3 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the parameters of q-ary RS code over GF(256) for dmin=33. | CO2 | 8 |
|  | b. | Consider the following binary sequences ABBABBAABBBAAABA. Use the Lempel – Ziv algorithm to encode this sequence. Assume that the binary symbols 1 and 0 are already in the code book | CO2 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Investigate on the ‘block preparation’ and quantization phases of JPEG compression process with a neat sketch. | CO3 | 8 |
|  | b. | Elucidate on the GIFF and TIFF image compression formats. | CO3 | 8 |
|  | c. | Write the drawbacks of Data compression. | CO3 | 4 |

ALL THE BEST