Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14EC2080 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | COMMUNICATION ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Illustrate how DSBFC –AM is generated by Square Law modulator and derive the expression to show that the output has all three components | CO2 | 15 |
|  | b. | Calculate the minimum height required of an antenna with frequency of 3Khz. | CO2 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Define Amplitude Modulation. Derive an expression for Amplitude Modulation with its power calculation and current relation. | CO3 | 15 |
|  | b. | Sketch the waveform of Phase Modulation(PM) and explain. |  | 5 |
| 3. | a. | Explain with a neat circuit diagram how AM –(DSBSC) signals are generated with the help of Balanced Modulator. | CO3 | 15 |
|  | b. | Compare Low level and High level modulation. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Construct a neat block diagram for AM Superhetrodyne receiver with required explanation. | CO2 | 10 |
|  | b. | Generate a Frequency Modulation (FM) expression using Direct Method with suitable block diagram. | CO3 | 10 |
| 5. | a. | Plan and prepare with a block diagram how Tuned Radio Frequency(TRF) Receiver works. | CO2 | 10 |
|  | b. | Write short notes on a)Pulse Width Modulation(PDM) b)Pulse Position Modulation (PPM) | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Illustrate about the circuit diagram of Envelope detector with necessary expressions. | CO3 | 10 |
|  | b. | Infer on the Errors that can be detected and corrected in a data transmission with its types. | CO3 | 10 |
| 7. | a. | Describe how (PCM) Pulse Code Modulation helps in transmitting the data. | CO2 | 10 |
|  | b. | Differntiate between Twisted pair and Coaxial cable. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Generate a circuit for Pre-emphasis and De-emphasis and explain with its application. | CO1 | 15 |
|  | b. | The noise figure of IF amplifier is 15dbs.The amplifier is preceeded by a pre-amplifier with again of 10dbs and noise figure of 6dbs.Find the overall noise figure. | CO1 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss with a neat block diagram about the scanning process of TV Transmitter and Receiver. | CO2 | 20 |

ALL THE BEST