Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April / May – 2017

| Code : | 14EC3019 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | DIGITAL SYSTEM AND ASIC DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Realize the following functions using a PLA: f1(x, y, z) = ∑ (0, 1, 2, 4) f2(x, y, z) = ∑(0, 5, 6, 7). | CO1 | 10 |
|  | b. | Implement the following function with a Multiplexer F(A,B,C,D) = Σm(0,1,3,4,8,9,15) | CO1 | 7 |
|  | c. | Explain the different types of Hazards. | CO1 | 3 |
| (OR) |  |  |  |  |
| 2. | a. | Design a Sequence Detector that produces output ‘1’ using Mealy machine whenever the Non-overlapping sequence 1011 is Detected. Draw the Synchronous sequential logic circuit for the same using D flipflop. | CO1 | 12 |
|  | b. | List out the differences between Latches and Flipflop. | CO1 | 3 |
|  | c. | Construct a 4 * 16 Decoder with two 3 * 8 decoders and explain its working. | CO1 | 5 |
| 3. | a. | Design the Synchronous sequential logic circuit for MOD 6 counter using JK flipflop. | CO1 | 10 |
|  | b. | Draw the PAL realization for the above circuit. (MOD 6 counter) | CO1 | 6 |
|  | c. | What is a Programmable logic device? Mention its advantages. | CO1 | 4 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the various Programming Technologies used in ASIC. | CO2 | 12 |
|  | b. | Explain the problems that occur in the Asynchronous Sequential circuits while designing. | CO1 | 5 |
|  | c. | What is meant by RTL synthesis? | CO2 | 3 |
| 5. | a. | List out the different types of ASIC. Explain briefly about Full custom and Semi custom ASIC. | CO2 | 15 |
|  | b. | Design a Sequence Detector that produces output ‘1’ using Moore machine whenever the sequence 1011 is detected using ASM chart. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the Actel ACT 1 architecture with neat diagram. And also explain the Actel ACT timing model with diagrams. | CO3 | 7 |
|  | b. | Draw the ASIC Design flow and briefly explain the process flow with the significance of each stage in the design flow. | CO2 | 10 |
|  | c. | Mention the different elements used in the ASM chart. | CO1 | 3 |
| 7. | a. | Explain the Xilinx XC3000 and XC5200 Configurable Logic Blocks with neat diagram. | CO3 | 12 |
|  | b. | List out the differences between the CPLD and FPGA. | CO3 | 2 |
|  | c. | Draw the Altera FLEX interconnect scheme and explain it’s working. | CO3 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the Halfgate ASIC Design in various design software tools? | CO2 | 8 |
|  | b. | Explain EDIF (Electronic Design Interchange Format) files for an inverter. | CO2 | 12 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. | a. | Explain the CPLD architecture with neat diagram. | CO3 | 8 |
|  | b. | Mention the difference between Hard Macrocells and Soft Macrocells in CPLD. | CO3 | 4 |
|  | c. | Explain CFI (CAD Framework Initiative) design with an example. | CO2 | 8 |