Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14EE2004 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTROMAGNETIC FIELDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the volume and surface area of a sphere of radius R using integration. | CO1 | 6 |
|  | b. | State Stoke’s Theorem. | CO1 | 4 |
|  | c. | A vector is given by . Find - The component of the vector x, y, z axis. - Magnitude of the vector - Unit vector of A. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Express the vector in the rectangular coordinate system. | CO1 | 10 |
|  | b. | Express the vector field in cylindrical coordinates at | CO1 | 7 |
|  | c. | Represent a point in terms of unit vectors along the coordinate axes. | CO1 | 3 |
| 3. | a. | Deduce the electric field intensity produced on the z axis, due to a circular ring conductor placed in the origin. | CO1 | 15 |
|  | b. | Given the vector field . Find at | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | An The electric field is expressed in rectangular coordinate system by E =6x2ax+6yay+4az. Find VMN if points M and N are specified by M (2, 6,-1 ) and N (-3, -3,2 ) | CO2 | 10 |
|  | b. | Derive Poisson equation from Gauss law and give its significance. | CO2 | 10 |
| 5. | a. | A circular current loop x2+y2=9, is in the z=0 plane and it carries a current of 3 Amps. Find the magnetic field at the points (0,0,4) and (0,0,-4). | CO2 | 6 |
|  | b. | Find the expression for magnetic field intensity due to a infinite long straight conductor carrying a current of I amperes at a point which has the distance of ‘a’ from the conductor. | CO2 | 10 |
|  | c. | Show the mathematical representation of Ampere’s Circuital Law. | CO2 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | A 200 turns rectangular coil 30 cm by 15 cm with a current of 5A is placed in a uniform magnetic field of 0.2 Tesla. Find the magnetic moment and the maximum torque. | CO2 | 6 |
|  | b. | Deduce the relationship between | CO2 | 4 |
|  | c. | A cylindrical capacitor consists of an inner conductor of radius ‘a’ and an outer conductor whose inner radius is ‘b’ . The space between the conductors is filled with a dielectric of permittivity and the length of the capacitor is L. Determine the capacitance of this capacitor. | CO2 | 10 |
| 7. | a. | A capacitor with air as the dielectric medium has a plate area of 1cm2 with a plate separation of 0.1mm.Find the displacement current and displacement current density for an applied voltage of 100sin(3.14*106 t). | CO2 | 10 |
|  | b. | Explain about displacement and conduction current and hence relate them with frequency. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Two dielectric materials are separated by a magnetic boundary. The permeabilities of medium I and medium II are µ1 and µ2 respectively. Deduce its boundary equations in tangential and normal form. | CO3 | 15 |
|  | b. | Derive the relationship between Jc and Jd in a time Harmonic Field. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Derive Maxwells equation in point form and in integral form. | CO3 | 20 |

ALL THE BEST