Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May– 2017

| Sub.Code: | 14EE2013 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub.Name | TRANSMISSION AND DISTRIBUTION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A generating station has the following daily load cycle . Draw the following i.) Load curve ii) Load Duration Curve iii) Maximum Demand iv) Units Generated/Day iv) Average Load v) Load Factor Time (Hrs); 0-6; 6-10; 10-12; 12-16; 16-20; 20-24. Load (MW); 40; 50; 60; 50; 70; 40. | CO1 | 10 |
|  | b. | Derive an expression for two phase three wire system. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | State the advantages of high Alternating Current (A.C) transmission voltage in a transmission system. | CO1 | 10 |
|  | b. | The daily demands of three consumers are given below: Time; Consumer 1; Consumer 2; Consumer 3. 12 midnight to 8 A.M; No Load; 200W; No load. 8 A.M to 2 P.M; 600 W; No Load; 200 W. 2 P.M to 4 P.M; 200W; 1000 W; 1200 W. 4 P.M to 10 P.M; 800 W; No Load; No Load. 10 P.M to midnight; No Load; 200 W. Plot the load curve and find i) maximum demand of individual consumer ii)load factor of individual consumer iii) diversity factor iv) load factor of the station | CO1 | 10 |
| 3. | a. | Derive an expression for inductance per phase for a 3-phase overhead transmission line when i) Conductors are symmetrically placed | CO1 | 10 |
|  | b. | ii)Conductors are unsymmetrically placed but the line is completely transposed. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | A 100-km long, 3-phase, 50-Hz transmission line has following line constants:Resistance/phase/km = 0·1 Ω, Reactance/phase/km = 0·5 Ω, Susceptance/phase/km = 10 × 10−6 S, If the line supplies load of 20 MW at 0·9 p.f. lagging at 66 kV at the receiving end, calculate by nominal π method. i) sending end power factor ii) regulation(iii) transmission efficiency | C02 | 10 |
|  | b. | Show how regulation and transmission efficiency are determined for medium lines using nominal T method. | CO2 | 10 |
| 5. | a. | In a three phase overhead system each line is suspended by a string of three insulators. The voltages at the top unit, near the tower and middle unit are 10kV and 11 kV respectively. a.Evaluate the ratio of shunt capacitance to self capacitance of each insulator. b.StringEfficiency c. Line Voltage. | CO2 | 12 |
|  | b. | What is corona loss? What are the factors affecting corona loss? How is it reduced? | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | Deduce an expression for sag in overhead lines when - Supports are at equal levels. - Supports are at unequal levels | CO2 | 8+7 |
|  | b. | An over head line has a span of 260m, the weight of the line conductor is 0.68 kg per metre run. Calculate the maximum sag in the line. The maximum allowable tension in the line is 1550 kg. | CO2 | 5 |
| 7. | a. | Derive for the dielectric stress in a single core cable. | CO2 | 10 |
|  | b. | Prove that gmax/gmin in a single core cable is equal to D/d where D is the internal sheath diameter and d is the core diameter. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | With the help of a near diagram explain the constructional details of a Single core low tension cable. | CO2 | 10 |
|  | b. | A single core lead sheathed cable is graded by using three dielectrics of relative permittivity 5, 4 and 3 respectively. The conductor diameter is 2 cm and overall diameter is 8 cm. If the three dielectrics are worked at the same maximum stress of 40 KV / cm, find the safe working voltage of the cable. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive an expression for AC distribution with respect to power factor referred to receiving end voltages. | CO3 | 10 |
|  | b. | At the conductor distributor AD is fed at A and D at 255 V and 250 V respectively and is loaded as shown. The resistance given are those of each conductor. Find the value of current in each section of the cable and the voltage at each load point. | CO3 | 10 |

ALL THE BEST

`