Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14EE2018 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENERGY SYSTEMS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Exemplifying the layout and governing of hydro power plant. | CO1 | 10 |
|  | b. | With neat diagram, explain the stand-alone photovoltaic system and also distinguish the stand-alone and grid interactive photovoltaic system. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Draw and explain the Horizontal Axis Wind Turbine (HAWT) and also comment on environmental impacts of wind energy. | CO1 | 10 |
|  | b. | Discuss the various methods of tidal power generation with neat sketches and also give the limitations of each method. | CO1 | 10 |
| 3. | a. | With help of neat diagram, explain the working of a central tower receiver power plant. | CO1 | 10 |
|  | b. | Discuss the necessity to develop the non-conventional methods of generating electrical energy. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | With help of an example, discuss the ten steps of detailed energy audit and also suggest the concluding remarks for your example in terms of energy conservation. | CO1 | 10 |
|  | b. | Discuss the fixed and running cost that go into the generation of electrical energy. | CO1 | 10 |
| 5. | a. | A power plant has to supply loads as follows: 12 PM to 5 AM; 500 kW. 5 AM to 10 AM; 800 kW. 10 AM to 12 Noon; 2000 kW. 12 Noon to 2 PM; 1000 kW. 2 PM to 5 PM; 2500 kW. 5 PM to 8 PM; 2000 kW. 8 PM to 10 PM; 1500 kW. 10 PM to 12 PM; 1000 kW. Determine the load factor and select a suitable number and size of units to supply this load. Reliability of supply is to be maintained. Determine the reserve capacity, the plant capacity factor, plant use factor and the operating schedule of the plant to meet the load. | CO1 | 10 |
|  | b. | Discuss the energy efficient lighting control methods. | CO1 | 5 |
|  | c. | List down the energy conservartion methods without investing the money. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss the function of photoemissive cell and describe its working when used alongwith a d.c amplifier. | CO2 | 10 |
|  | b. | With neat diagram, describe the method of measuring brightness of a light source. | CO2 | 10 |
| 7. | a. | A 20 kW single phase, 220 V resistance oven employees circular nichrome wire for its heating element. If the wire temperature is not to exceed 1127° C and the temperature of the charge is to be 427° C, calculate the size and length of wire required. Assume e=0.9 and radiation efficiency K=0.6. Determine the temperature of wire when the charge is cold. Assume ρ=1.09 x 10-6 Ω-m. | CO2 | 10 |
|  | b. | Explain with neat sketches, the construction, principle of operation and application of Ajax Wyatt furnace. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Name and describe various resistance welding processes with neat diagrams. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the expression for tractive effort during acceleration in mechanism of train movement. | CO2 | 15 |
|  | b. | Illustrate rheostaic braking and regenerative braking are obtained from single phase series motors. | CO2 | 5 |

ALL THE BEST