UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – April / May – 2017

| Code : | 14EE2026 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HIGH VOLTAGE ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | Explain with neat diagrams two different theories of charge generation and separation in a thunder cloud. | CO 1 | 10 |
|  | b. |  | What is a travelling wave? Explain the development of such a wave on an overhead line. | CO 1 | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. |  | Explain with neat sketches the mechanism of lightning discharge. | CO 1 | 10 |
|  | b. |  | What are ground rods and counterpoises? Explain clearly how these can be used to improve the grounding conditions. Give various arrangements of counterpoise. | CO 1 | 10 |
| 3. | a. |  | Define Townsend’s first and second ionization coefficients. Explain the Townsends criterion for a spark. | CO 1 | 10 |
|  | b. |  | What are ‘Treeing’ and ‘Tracking’? Explain clearly the two processes in solid dielectrics. | CO 1 | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. |  | State and explain Paschen’s law. How do you account for the minimum voltage for breakdown under a given pd condition? | CO 3 | 10 |
|  | b. |  | Explain briefly various theories of breakdown in liquid dielectrics. | CO 3 | 10 |
| 5. | a. |  | Explain and compare the performance of half wave rectifier and voltage doubler circuits for generation of high d.c. voltages. | CO 2 | 10 |
|  | b. |  | What is a cascaded transformer? Explain why cascading is done? Describe with neat diagram a three stage cascaded transformer. Label the power ratings of various stages of the transformer. | CO 2 | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. |  | A ten-stage impulse generator has 0.250 μF condensers. The wave front and wave tail resistances are 75 ohms and 2600 respectively. If the load capacitance is 2.5 nF, determine the wave front and wave tail times of the impulse wave. | CO 3 | 10 |
|  | b. |  | Define the terms (i) Impulse voltages; (ii) Chopped wave; (iii) Impulse flash over voltage; (iv) Impulse puncture voltage; (v) Impulse ratio for flash over; (vi) Impulse ratio for puncture. | CO 3 | 10 |
| 7. | a. |  | Explain with neat diagram the principle of operation of an Electrostatic Voltmeter. Discuss its advantages and limitations for high voltage measurements. | CO 2 | 10 |
|  | b. |  | Discuss various resistance potential dividers and compare their performance of measurement of impulse voltages | CO 2 | 10 |
|  | (OR) |  |  |  |  |
| 8. | a. |  | Discuss various capacitance, potential dividers and compare their performance for measurement of impulse Voltages | CO 2 | 10 |
|  | b. |  | Draw Chubb-Fortescue Circuit for measurement of peak value of a.c. voltages discuss its advantages over other methods. | CO 2 | 10 |

|  |  | Compulsory: |  |  |
| --- | --- | --- | --- | --- |
| 9. | a. | List out various tests to be carried out on insulator and give a brief account of each test. | CO 3 | 10 |
|  | b. | Describe various tests to be carried out on C.B. | CO 3 | 10 |