Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Question Paper – Apr/May – 2017

| Code : | 14EE3014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | WIND ENERGY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the expression for power contained in wind and highlight the significance of the wind turbine power curve. | CO1 | 10 |
|  | b. | A 2.4 MW wind turbine has an installed cost of $2.4 x 106. Assume that the annual operating and maintenance costs are 2% of the initial installed cost. The wind regime at this site results in a capacity factor (CF) of 0.35. It is also assumed that the lifetime of this turbine is 25 years and that the electrical output can be sold for $0.05/ kWh. For a 5% discount rate calculate the following economic parameters for this system: i) Average annual return ii) Simple payback period iii) The net present value of the savings (NPVS) iv) The benefit-cost ratio (B/C) v) The levelized cost of energy (COEL) | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Based on average speed data only, estimate the annual energy production from a horizontal axis wind turbine with a 40 m diameter operating in a wind regime with an average wind speed of 12 m/s. Assume that the wind turbine is operating under standard atmospheric conditions (ρ = 1.225 kg/m3). Assume a turbine efficiency of 0.6. | CO1 | 10 |
|  | b. | Discuss in detail the value of wind energy and the significance of economic assessment of wind energy. | CO1 | 10 |
| 3. | a. | From an analysis of wind speed data (hourly interval average, taken over a one year period), the Weibull parameters are determined to be c = 6 m/s and k = 1.8. i) What is the average velocity at this site? ii) Estimate the number of hours per year that the wind speed will be between 6.5 and 7.5 m/s during the year. iii) Estimate the number of hours per year that the wind speed is above 16 m/s. | CO1 | 6 |
|  | b. | A 40 m diameter wind turbine is placed on a 100 m tower in terrain with a power law coefficient (α) of 0.15. Find the ratio of available power in the wind at the highest point the rotor reaches to its lowest point | CO1 | 4 |
|  | c. | Many researchers have noted that a major barrier to deployment in many regions of the world is a lack of reliable and detailed wind resource data. Discuss in detail the different methodologies of wind resource assessment. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | A wind farm is being considered for a ridge top site. Name 10 or more issues that should be considered in evaluating this site. | CO1 | 10 |
|  | b. | The atmospheric boundary layer is the lowest part of the atmosphere and its characteristics are directly influenced by contact with the earth’s surface. Discuss in detail the characteristics of atmospheric boundary layer and its impact on the wind resource | CO1 | 10 |

| 5. | a. | Review in detail about Wind Measurement and Instrumentation. | CO1 | 10 |
| --- | --- | --- | --- | --- |
|  | b. | Write down the procedure for generalized rotor design of a wind turbine using appropriate expressions. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | i) Find θ, θP, θT, and c for one blade section from r/R = 0.45 to r/R = 0.55 (centered on r/R = 0.50) for an ideal blade (assume Cd = 0, a′ = 0). Assume λ =7, B=3, R=5 m, and Cl =1.0 and the minimum Cd / Cl occurs at α = 7. ii) Assume that Cd / Cl actually equals 0.02 for the above blade section and that the free stream wind speed, U, equals 10 m/s. Find Urel , dFL1 , dFD1 , dFN1 , dFT1, dQ1 for the blade section. Don't forget to consider that the wind velocity is slowed down at the rotor. Use a = 1/3, a′ = 0. Assume the air density is 1.24 kg/m3 (20 °C). | CO2 | 10 |
|  | b. | Derive expressions for the rotor blade shape using strip theory including wake rotation. | CO2 | 10 |
| 7. | a. | India’s first ever offshore wind power project is being set up in which state? Discuss the general considerations for offshore wind energy, offshore wind turbines and their support structures. Highlight the significance of metocean design conditions. | CO1 | 10 |
|  | b. | Two wind turbines emitting 105 dB(A) at the source are located 200m and 240m away from a location of interest. Calculate the sound pressure level (in dB(A)) at the point of interest from the combined acoustic effect of the two turbines. Assume a sound absorption coefficient of 0.005 dB(A)/m. | CO2 | 5 |
|  | c. | Wind turbine blades use airfoils to develop mechanical power. Discuss with a neat diagram the airfoil terminology and the forces and moments on an airfoil section. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | A small wind turbine generator (single phase) produces a 60 Hz voltage at 120 V rms. The output of the generator is connected to a diode bridge full-wave rectifier, which produces a fluctuating DC voltage. What is the average DC voltage? A silicon-controlled rectifier (SCR) is substituted for the diode rectifier. Under one condition the SCRs are turned on at 60 deg after the beginning of each half cycle. What is the average DC voltage in that case? | CO3 | 3 |
|  | b. | Highlight the different stand-alone configuration of wind energy generating systems | CO3 | 7 |
|  | c. | To successfully generate power using various components, wind turbines need a control system that ties the operation of all the subsystems together. With neat diagrams, discuss in detail the wind turbine control systems. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | An isolated power system serving the community in the outskirts of Sadivayal uses a 100 kW diesel generator. The community plans to add 50 kW of wind power. The hourly average load and power from the wind turbine over a 24 hour period is detailed in Table given. The diesel generator has a no-load fuel usage of 4 liter per hour and an additional incremental fuel use of 1/4 liter per kilowatt-hour. For the day in question, using the hybrid system design rules, determine: i) The maximum renewable energy that can possibly be used in an ideal system. ii) The maximum renewables contribution without storage, and the maximum with storage. iii) The maximum fuel savings that can be achieved. iv) The minimum diesel fuel use that can be achieved with intelligent use of storage and controls. Table : Diesel System Load data Hour; Load (kW); Wind (kW); Hour; Load (kW); Wind (kW). 0; 30; 40; 12; 95; 40. 1; 25; 40; 13; 75; 50. 2; 20; 30; 14; 85; 50. 3; 25; 32; 15; 80; 45. 4; 30; 25; 16; 90; 34. 5; 40; 20; 17; 76; 56. 6; 50; 21; 18; 67; 37. 7; 30; 24; 19; 85; 50. 8; 60; 32; 20; 64; 50. 9; 50; 45; 21; 43; 45. 10; 60; 34; 22; 40; 40. 11; 40; 20; 23; 35; 41. | CO3 | 10 |
|  | b. | Write a note on the ancillary electrical equipments associated with a wind turbine installation. | CO3 | 10 |

ALL THE BEST