Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – APRIL/MAY – 2017

| Code : | 14EE3038 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | POWER QUALITY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Classify and illustrate the typical power quality phenomena defined in IEEE 1159. | CO2, CO1 | 10 |
|  | b. | Draw the CBEMA & ITI curve and explain about events described in the curves. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Examine the source and effects of different categories of long duration voltage Variations. | CO2 | 10 |
|  | b. | Give outlines on IEEE and IEC power quality standards. | CO3 | 10 |
| 3. | a. | In a three-phase system, the load voltage consists of Va=0.9sinωt, Vb=0.8sin(ωt-120), Vc=0.7sin(ωt-240). Calculate the following quantity with a sag period of 10ms, 20ms, 30ms with respect to ABC phases. i) Detroit Edison Sag Score (SS). ii) Voltage sag Energy. iii) Voltage sag lost energy index (VSLEI). | CO2,CO1 | 8 |
|  | b. | Explain the different types of sag with a relevant vector diagram with proper equations. | CO2, CO3 | 12 |
| (OR) |  |  |  |  |
| 4. |  | Explain the following causes of sags. a) Voltage sag due to motor sag. b) Voltage sag due to single line to ground fault. c) Voltage sag due to transformer energizing. | CO2, CO3 | 20 |
| 5. |  | Give outlines on sources of over voltage due to the following phenomenon a) Capacitor switching. b) Lightning. | CO2, CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | What is the need for protection against over voltages? What are the basic principles of over voltages protection of load equipments? | CO2, CO1 | 10 |
|  | b. | Explain in detail about surge arrestors and surge suppressors for over voltage Protection. | CO2, CO1 | 10 |
| 7. | a. | The converter circuit is shown the below fig. Calculate the following quantities (i)THD of AC mains current (ii) DIN (iii) TDD of AC mains current (iv) DF (v)DPF (vi) PF | CO2, CO3 | 12 |
|  | b. | Discriminate between linear and non linear load. | CO2 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Explain in detail about general procedure for harmonic distortion evaluation at the point of coupling. | CO2, CO3 | 10 |
|  | b. | Discuss the construction and working principle of active filters for harmonic mitigation. | CO2, CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the need and the functioning of power quality analyser. | CO2, CO3 | 10 |
|  | b. | Explain the following - Harmonic Analyser - Flicker Meter | CO2, CO3 | 10 |

ALL THE BEST