Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14FP2005 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HEAT AND MASS TRANSFER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive an expression for heat transfer in conduction through composite wall rectangular wall (series). | CO1 | 8 |
|  | b. | The maximum tolerable heat loss through a furnace wall is1.3 kw/m 2. A brick wall is constructed next to the furnace wall to insulate the heat loss. The temperature on either side of the wall is 200°C and 40 °C. What should be the thickness of the brick wall if the thermal conductivity of the brick is 0.72 W/m°C ? | CO2 | 8 |
|  | c. | Write a note on thermal resistance concept. | CO3 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Compare and contrast Natural and forced convection. | CO1 | 4 |
|  | b. | Estimate the convective heat-transfer coefficient for convective heat loss from a horizontal 10 cm diameter steam pipe. The surface temperature of the uninsulated pipe is 130°C, and the air temperature is 30°C. Density 0.968 kgm-3, β = 2.83 x 10-3K-1, ∆T= 100 K, Cp = 1.019 kJkg-1K-1, Npr = 0.71, k = 0.0293 Wm-1K-1, µ = 20.79 X10-6 Nsm-2 | CO2 | 8 |
|  | c. | Explain in detail about 1. Nucleate boiling 2. Film boiling 3. Leidenfrost point. | CO3 | 8 |
| 3. | a. | Explain in detail about Thermal boundary and Hydrodynamic boundary layers. | CO2 | 8 |
|  | b. | Sucrose syrup flows in a pipe of 0.023 m inside diameter at a rate of 40 lt/min, while steam is condensing on the outside surface of the pipe. The syrup is heated from 50 to 70°C, while the inside wall temperature is at 80°C. Calculate: 1) the heat transfer coefficient and 2) the required length of the pipe. ρ60°C=1200 kgm-3; μ60 = 3.8 cP; μ80 = 2.3 cP; CP@60°C= 3120 J/Kg°C; k@60°C = 0.46 W/m°C. |  | 8 |
|  | c. | Write a note on 1. Laminar flow and 2. Turbulent flow. | CO1 | 4 |
| (OR) |  |  |  |  |
| 4. | a. | Write in detail about convection heat transfer. | CO3 | 8 |
|  | b. | Enumerate the steps for calculation of convective heat transfer coefficients using empirical correlations. | CO2 | 8 |
|  | c. | What is the expected percent increase in convective heat-transfer coefficient if the velocity of a fluid is doubled while all other parameters are kept the same for turbulent flow in a pipe? | CO1 | 4 |
| 5. | a. | State Stefan-Boltzmann’s law of radiation and Kirchhoff’s law of radiation. | CO1 | 8 |
|  | b. | Show that a black body is a good emitter as well as absorber. | CO2 | 8 |
|  | c. | Differentiate thermal radiation from conduction and convection. | CO1 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | In a canning process, 100 cans per minute are to be heated from 60°C to 100°C, and demoistured of the adhering moisture to the cans. Each can contains on average 0.3m1 of water adhering to it. The cans have an emissivity of 0.7 and an average area of each can is 0.047 m2 , and its weight is 100g. The specific heat of can material is 0.25 kJ/kg°C. Calculate the temperature of the oven considering the radiative heat transfer only. | CO2 | 15 |
|  | b. | List the principles of thermal radiation. | CO1 | 5 |
| 7. |  | A liquid food (specific heat 4.0 kJ/kg °C) flows in the inner pipe of a double pipe heat exchanger. The liquid food enters the heat exchanger at 20°C and exits at 60°C .The flow rate of the liquid food is 0.5 kg/s. In the annular section, hot water at 90°C enters the heat exchanger and flows counter currently at a flow rate of 1 kg/s. The average specific heat of water is 4.18 kJ/(kg°C). Assume steady-state conditions. 1. Calculate the exit temperature of water. 2. Calculate log-mean temperature difference. 3. If the average overall heat transfer coefficient is 2000 W/(m2°C) and the diameter of the inner pipe is 5cm, calculate the length of the heat exchanger. 4. Repeat these calculations for parallel flow configuration. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A flow of 9.2 kg/sec of milk is to be heated from 15°C to 65°C in a heat exchanger using 16.7kg/s of water at 95°C. If overall heat transfer co-efficient is 1300 J/m2s °C , Calculate area of heat exchanger required if the flow rates are i) parallel and ii) Counter. Data: Sp. Heats of milk and water are 3890 and 4180 J/Kg °C. respectively. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe in detail about principle and theory of diffusion. | CO2 | 15 |
|  | b. | State Fick’s law of diffusion. | CO3 | 5 |

ALL THE BEST