Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Apr/May – 2017

| Code : | 14FP2007 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-I | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the working of cabinet dryer with a neat figure. | CO2 | 10 |
|  | b. | Determine the total time required for drying period for the data given below. The food material is dried from 26 to 7% moisture content (wet basis) Weight of dry solids (Wd) = 180 kg. Weight of dry solids per unit drying area = 20 kg / m2. Moisture content at the end of constant rate period(XC)=0.25 kg moisture/kg of dry solids. Rate of drying at the end of constant rate period (RC) = 1.541 kg/m2 hr. Rate of drying at the end of falling rate period (R2) = 0.085 kg/m2 hr. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the working of deep bed dryer with diagram. | CO1 | 10 |
|  | b. | Determine the equilibrium moisture content of sorghum at RH = 10 % and T=60℃ using Henderson’s equation where c = 6.12 x 10-6 and n = 2.31. | CO3 | 10 |
| 3. | a. | Illustarte the principle and working of rotary vacuum dryer with a neat sketch. | CO2 | 15 |
|  | b. | A tubular bowl centrifuge is used for separation of cream from milk which has the discharge diameters of 12 cm and 16 cm for the solids and liquid respectively. If the density of milk is same as that of water and the cream density is equal to 920 kg/m3, calculate the radius of the neutral zone. | CO3 | 5 |
| (OR) |  |  |  |  |
| 4. |  | Derive the formula for the following during the separation of liquids in a centrifuge: - Pressure acting in the centrifuge due to the liquid(ΔP). - Radius of neutral zone (rn). - Residence time of the particle in the centrifuge (tr). - Volumetric flow rate from the centrifuge (Q). | CO2 | 20 |
| 5. | a. | Draw a neat figure of forced circulation evaporator indicating the various components and demostrate the working process. | CO1 | 10 |
|  | b. | Milk containing 3.7% fat and 12.8% total solids is to be evaporated to produce a product containing 7.9% fat. Determine the mass of the product from 100 kg of milk and the total solids concentration in the final product, assuming that there are no losses during the process. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. |  | Demonstrate the different types of feeding methods in triple effect evaporators with suitable figures. | CO2 | 20 |
| 7. | a. | Sketch the figure of disc attrition mills and enumerate the working. | CO2 | 10 |
|  | b. | Sugar crystals were ground from an average diameter of 500 µm to powder with an average diameter of 100 µm. The net energy consumption was 0.5 kWh per tonne. Calculate the net energy consumption for grinding the crystals to 50 µm powder. Apply Rittinger’s and Kick’s laws. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the working of hammer mill with suitable diagram. | CO2 | 10 |
|  | b. | Food grains of average initial size 5 mm were reduced to 0.0014 mm size using a 10 hp motor. Check whether this motor would be adequate to reduce the size of the particles to 0.0006 mm? Assume Rittinger’s equation and 1 hp = 745.7 watts. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the principle and working of ribbon mixer with a neat diagram. | CO2 | 10 |
|  | b. | During the preparation of a dough, 1 kg of sugar was mixed with 99 kg of flour in a sigma mixer. Three samples were taken after 10 minutes of mixing and analysed for the percentage of sugar in the mixture. The results obtained were : 0.78, 0.69 and 0.72. Calculate the mixing index and the time required for perfect mixing. Consider σα = 0.01 | CO3 | 10 |

ALL THE BEST