Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April / May – 2017

| Code : | 14FP2010 | Duration : | 3 hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS IN FOOD PROCESS ENGINEERING-II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write a note on Raolt’s law and relative volatility with suitable formula. | CO1 | 10 |
|  | b. | Calculate the relative volatility for benzene-toluene syatem at 85℃ and 105℃ for the given data: At 85℃, PA = 116.9 kPa and PB = 46 kPa and at 105℃ PA = 204.27 kPa and PB = 86.2 kPa | CO2 | 5 |
|  | c. | Write the applications of distillation in food processing. | CO3 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain steam distillation process with a neat sketch. | CO2 | 10 |
|  | b. | Steam distillation process is used for separating out a spice oil from the spices by passing steam into the chamber at 50 kPa at the rate of 240 kg/hour for 20 minutes. If the molecular weight of the volatile oil and steam are 165 and 18 respectively, find the quantity of spice distilled. Consider the total pressure as 101.3 kPa. | CO3 | 10 |
| 3. | a. | Explain the multi stage extraction process with diagram. | CO2 | 10 |
|  | b. | Give a detailed note on the applications of extraction in food processing. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the working of continuous counter current extractor with neat diagram. | CO2 | 10 |
|  | b. | The natural food colour from carrot is to be extracted with water. The saturated concentration of the colour in the water is found to be 3 kg/m3. In a laboratory scale extractor containing about one litre volume, it has taken 20 minutes to extract the colour from beet root to an extent of 0.985 kg/m3. Under similar conditions in a commercial plant of 10 m3 capacity, it is desired to extract 12 kg of the colour into the water. How much time does it take? | CO2 | 10 |
| 5. | a. | Explain the working principle and functioning of draft tube baffle crystallizer with a sketch. | CO1 | 15 |
|  | b. | Enlist the various applications of crystallization in food processing. | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Describe a batch type crystallizer with a neat diagram write its application in food industries. | CO2 | 10 |
|  | b. | Find the nucleation rate of sodium chloride crystals at 20℃ which have a surface tension of 6.02 ergs/cm2. The solution has a fractional super saturation(S) of 0.025. The density of NaCl is 2163 kg/m3 and molecular weight of NaCl is 58.45. | CO2 | 10 |
| 7. | a. | Explain the mass transfer process in microfiltration membranes. | CO2 | 10 |
|  | b. | A membrane for microfiltration was examined microscopically and found to have about 2,40,000 pores with an average diameter of 0.75 x 10-6 m per 2 mm2 of membrane surface. The thickness of the membrane is 170 x 10-6 m. The viscosity of the permeate is 0.0014 Pa.s. Estimate the following : a) Porosity of the membrane (€), b) Hydraulic Permeability (LP) and c) Permeate Flux(J) for a Trans membrane Pressure Difference(∆PTM) of 1.35 Pa. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the solvent transport process in reverse osmosis and applications of the process. | CO2 | 15 |
|  | b. | Estimate the osmotic pressure of orange juice with 13% total solids at 20℃. The density of orange juice is 1048 kg/m3, Gas constant = 8.314 (m3 kPa/kg mol K) and molecular weight is 180 kg/kg mol. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the working of packed towers for gas absorption. | CO2 | 15 |
|  | b. | Enumerate the applications of gas absorption in food processing. | CO3 | 5 |