Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14MA3002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FINITE ELEMENT METHODS | Max. marks : | 100 |

ANSWER ALL QUESTIONS  5 x 20 = 100 Marks

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write a brief note on Mathematical Models. | CO1 | 5 |
|  | b. | Derive the harmonic motion of the simple pendulum. | CO1 | 15 |
| ( OR) |  |  |  |  |
| 2. | a. | Expalin the steps of finite element methods. | CO1 | 5 |
|  | b. | Construct the governing equation for the axial deformation of a bar of variable cross section and find its solution. | CO1 | 15 |
| 3. | a. | Find the force-displacement relation of a linear elastic spring. | CO3 | 10 |
|  | b. | Discuss the discrete problem of torsion of circular shafts. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Discuss the governing equations of conduction heat transfer in fins, plane wall and cylindrical system. | CO3 | 20 |
| 5. | a. | Write a brief note on Eigen value problems. | CO3 | 5 |
|  | b. | Construct the Eigen value problem to find the axial motion of a bar and discuss its solution. | CO3 | 15 |
| (OR) |  |  |  |  |
| 6. |  | Formulate the Eigen value problem to find the transient response of a parabolic equation and find its solution. | CO3 | 20 |
| 7. | a. | Dervie the quadrature formulas for integrals over a rectangular master element. | CO2 | 10 |
|  | b. | Evaluate ∂ψi∂x and ∂ψi∂yfor the quadrilateral element using isoparametric formulation. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Compute S1100 and S1211 for the quadrilateral element using Gauss-Legendre quadrature. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the Hamilton’s principle for an elastic body. | CO2 | 10 |
|  | b. | Find the Euler equation of the function u x . | CO2 | 10 |

ALL THE BEST