Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14ME2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENGINEERING MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the conditions of equilibrium? | CO1 | 5 |
|  | b. | Two cables are tied together at C and are loaded as shown. Determine the tension (i) in cable AC, (ii) in cable BC. Where α is 20°. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Four forces act on bolt A as shown. Determine the resultant of the forces on the bolt. | CO1 | 15 |
|  | b. | State and prove parallelogram of forces. | CO1 | 5 |
| 3. | a. | The tension in cables AB and AC are 100N and 120N respectively as shown in figure below. Determine the magnitude of the resultant force acting at A. | CO1 | 15 |
|  | b. | Write short notes on free body diagram. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | A 4.8m beam is subjected to the forces shown in fig. Reduce the given system of forces to a) a single force b) an equivalent force- couple system at A c) force couple system at B | CO1 | 15 |
|  | b. | State varignons theorem. | CO1 | 5 |
| 5. |  | Find the moment of inertia of a unsymmetrical I section about its centroid axes having top flange as 60mm *20mm, bottom flange as 100mm*20mm and web is 60mm*20mm. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Locate the moment of inertia of the sectioned area shown in figure. | CO2 | 20 |
| 7. | a. | A horizontal force P= 600N is exerted on block A of mass 120kg as shown in fig. The co.efficient of friction between the block A and the horizontal plane is 0.25. Block B has a mass 30kg and the Co.efficient of friction between it and the plane is 0.4. The wire between the two blocks makes 30° with the horizontal. Calculate the tension in the wire. | CO3 | 15 |
|  | b. | A bullet of mass 30 gram is moving horizontally with a velocity of 450 m/s and strike a wodden block of weight 45 N, resting on a horizontal floor. The bullet is embedded into the block and then both block and bullet move as a single unit. Calculate the distance moved ?. μ = 0.45 . | CO4 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Two blocks A (mass 28kg), B (mass 28 kg) are separated by 12 m as shown in the figure. If the blocks start moving, find the time t when the blocks collide. Assume µ = 0.25 for block A and plane and µ = 0.1 for block B and plane. | CO3 | 15 |
|  | b. | State and prove parallel axis theorem. | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | A uniform ladder weighing 100N and 5 meters long has lower end B resting on the ground and upper end A resting against a vertical wall as shown in fig.the inclination of the ladder with horizontal is 60°.if the coefficient of the friction at all surfaces of contact is 0.25, determine how much distance up long the ladder a man weighing 600N can ascent without causing it to slip. | CO4 | 20 |

ALL THE BEST