Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April / May – 2017

| Code : | 14ME2016 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | THERMAL ENGINEERING II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the pressure Lubrication system with a neat sketch. | CO1 | 10 |
|  | b. | Explain with neat sketch the valve timing diagram of 4 stroke diesel engine. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A four stroke single cylinder in an IC engine develops 5 kW of brake power with a mechanical efficiency of 80 % running at 1500 rpm. If the indicated mean effective pressure is 7 bar, find the dimensions of the cylinder assuming stroke length and diameter are equal. | CO1 | 10 |
|  | b. | A petrol engine develops 20 kW of brake power. The brake thermal and mechanical efficiencies of the engine are 25% and 80 %, respectively. Calculate the indicated thermal efficiency. | CO1 | 10 |
| 3. |  | Derive an expression for the air standard efficiency of Otto cycle. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | An ideal diesel cycle has compression ratio of 16 with lowest temperature and pressure as 27C and 1 bar respectively. If Heat added during the cycle is 500kJ/kg of air, find pressure and temperature at each state point, work done in the cycle/kg, thermal efficiency of the cycle and mean effective pressure. | CO1 | 20 |
| 5. |  | Explain summer air-conditioning system for hot and wet weather with a neat sketch. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A conference room for seating 100 persons is to be maintained at 220C dry bulb temperature and 60% relative humidity. The outdoor conditions are 400C dry bulb temperature and 270 C wet bulb temperatures. The various loads in the auditorium are as follows: Sensible heat load per person = 80 W Latent heat load per person = 50 W Lights and fans =15000 W Sensible heat gain through glass, walls, ceilings etc. =15000 W. The air infiltration is 20 m3/min. Determine the room sensible heat factor (RSHF). | CO4 | 20 |
| 7. |  | A nozzle in a wind tunnel gives a test section Mach number of 2.0. Air enters the nozzle from a large reservoir at 0.69 bar and 310K. The cross sectional area of the throat is 1000 cm2. Determine the following quantities for the tunnel for one dimensional isentropic flow: a) Pressures, temperatures and velocities at the throat and test sections. b) Area of cross section of the test section. b) Mass flow rate c) Power required to drive the compressor. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A circular duct passes 8.25 kg/s of air at an exit Mach number of 0.5.The entry pressure and temperature are 3.45 bar and 380C respectively and the coefficient of friction is 0.005.If the Mach number at entry is 0.15. Determine a. The diameter of the duct b. Length of the duct c. Pressure and temperature at the exit, d. Stagnation pressure loss and e.Verify the exit Mach number through exit velocity and temperature. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | With neat sketch explain the constructional features and working principle of Ramjet Engine. Write its advantages and disadvantages. | CO3 | 20 |