Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14ME2019 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | HEAT AND MASS TRANSFER | Max. marks : | 100 |

(Permitted to use Heat and Mass Transfer Data Book)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the general three-dimensional heat conduction equation in Cartesian coordinates. | CO1 | 10 |
|  | b. | An exterior wall of a house may be approximated by a 0.1m layer of common brick (k=0.7 W/m°C) followed by a 0.04m layer of gypsum plaster (k=0.48 W/m°C). What thickness of loosely packed rock wool insulation (k=0.065 W/m°C) should be added to reduce the heat loss or (gain) through the wall by 80 percent ? | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | An electric current is passed through a plane wall of thickness 25 mm and 120 mm width, which is used to heat a fluid at 30oC. The heat generation rate is 65 x 105 W/m3 ,thermal conductivity of the plate is 25 W/mK. Calculate the heat transfer co-efficient to maintain the temperature of the plate below 150oC. | CO1 | 10 |
|  | b. | A wire of 6 mm diameter with 2 mm thick insulation (K = 0.11W/mK). If the convective heat transfer co-efficeint between the insulating surface and air is 25 W/m2K, find the critical thickness of insulation. And also find the percentage of change in the heat transfer rate if critical radius is used. | CO1 | 10 |
| 3. | a. | Consider aluminium sphere weighing 7 kg and initially at a temperature of 320oC is suddenly immersed in a liquid at 25oC. The convective heat transfer co-efficient is 50 W/m2K. Estimate the time required for the sphere to reach 100oC, using the lumped theory. | CO1 | 15 |
|  | b. | A pipe 600mm in diameter carrying steam at a temperature of 180oC (for heating the house in winter) and another pipe 180mm diameter carrying water at 12oC (for supplying drinking water) are buried under the ground with centre to centre distance of 1.8m.The length of each pipe is 90m and thermal conductivity of soil is 0.45W/m oC.Assuming that the pipes run parallel to each other,the earth is infinite and resistance of the pipe material is negligible. Determine the net heat transfer rate between the two pipes. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Define Fourier and Biot numbers and mention its physical significance. | CO1 | 5 |
|  | b. | A long steel cylinder 12 cm in diameter and initially at 20 oC is placed into a furnace at 820 oC with the local heat transfer coefficient h=140W/m2 K.Calculate the time required for the axis temperature to reach 800 oC.Also calculate the corresponding temperature at a radius of 5.4cm at that time.The physical properties of steel are α=6.11x10-6 m2/s,k=21W/m K. | CO1 | 15 |
| 5. | a. | Air at 20oC and at atmospheric pressure flows over a flat plate at a velocity of 3 m/s. If the plate is 1 m wide and 80oC, calculate the following at x = 300 mm from the leading edge. a) Hydrodynamic boundary layer thickness b) Thermal boundary layer thickness c) Local friction coefficient d) Average friction coefficient e) Local heat transfer coefficient f) Average heat transfer coefficient and g) Heat transfer. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Water flows inside a tube of 20 mm diameter and 3 m long at a velocity of 0.03 m/s. Water gets heated from 40oC to 120oC while passing through the tube. The tube wall is maintained at constant temperature of 160oC. Find heat transfer | CO1 | 10 |
|  | b. | A stainless steel cylindrical rod fin of 1.2 cm diameter and 6 cm height with thermal conductivity of 25 W/mK is exposed to surrounding with a temperature of 60oC. The heat transfer co-efficient is 45 W/m2K and the temperature at the base of the fin is 100oC. Determine fin efficiency, temperature at the edge of the rod and heat dissipation.Assume fin end is insulated. | CO1 | 10 |
| 7. | a. | In a counter flow double pipe heat exchanger, oil is cooled from 85oC to 55oC by water entering at 25oC. The mass flow rate of oil is 9800kg/h and specific heat of oil is 2000 J/kg K. The mass flow rate of water is 8000 kg/h and specific heat of water is 4180 J/kg. Determine the heat exchanger area and heat transfer rate for an overall heat transfer co-efficient of 280 W/m2 K. | CO2 | 15 |
|  | b. | Discuss the advantage of NTU method over the LMTD method. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Estimate the rate of diffusion of water vapour from a pool of water at the bottom of a well which is 6.2m deep and 2.2m diameter to dry ambient air over the top of the well.The entire system may be assumed at 30 oC and one atmosphere pressure.The diffusion co-efficient is 0.24x10-4m2/s. | CO3 | 10 |
|  | b. | CO2 and air experience equimolar counter diffusion in a circular tube whose length and diameter are 1.2 m and 60 mm respectively. The system is at total pressure of 1 atm and at a temperature of 273K. The ends of the tube are connected to large chambers.The partial pressure of CO2 at one end is 2000 mm of Hg while at the other end is 90 mm of Hg. Estimate the mass transfer rate of CO2 and air through the tube. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Two large paralle plates with emmisivity of 0.5 each are maintained at different temperatures and are exchanging heat only by radiation. Two equally large radiation shields with surface emmisivity 0.05 are introduced in parallel to the plates. Find the percentage reduction in net radiative heat transfer. | CO1 | 15 |
|  | b. | Briefly explain the concept of black body . | CO1 | 5 |

ALL THE BEST