Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April / May – 2017

| Code : | 14ME2029 | Duration : | 3 hrs |
| --- | --- | --- | --- |
| Sub. Name : | DESIGN OF MACHINE ELEMENTS | Max. marks : | 100 |

Note: Use of DESIGN DATA book & Approved data sheets are permitted. Necessary data may be suitably assumed.

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define Engineering Design.list the steps involved in asystematic design Process. | CO1 | 6 |
|  | b. | An overhang crank with pin and shaft is shown in Fig . A tangential load of 15 kN acts on the crank pin. Determine the maximum principal stress and the maximum shear stress at the centre of the crankshaft bearing. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the material properties: hardness, stiffness and resilience. | CO1 | 6 |
|  | b. | A steel bracket is loaded as shown in the Fig 1. The section at A-A is rectangular whose width is 40 mm and thickness‘t’ mm. The permissible tensile stress of the material of the bracket is 60 N/mm2. Determine the thickness of the bracket. Fig 1 | CO1 | 14 |
| 3. | a. | How the springs are classified? Mention the application of each type. | CO1 | 6 |
|  | b. | Design a semi-elliptical leaf spring for the rear axle of a car . The load on the rear axle of the car is 10 kN. The span is 1 m and the width of the clamp is 100 mm. In all, 12 leaves are used, out of which two are main leaves and the remaining are graduated leaves. The permissible stress is 300 N/mm2. The ratio of total depth to width of the spring is 2. Find the dimensions of the leaf spring, deflection and the length of all the leaves. Assume E = 2 X 105 N/mm2 | CO1 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | What do you understand by preferred numbers? | CO1 | 4 |
|  | b. | A line shaft running at 1000rpm transmits 20kW.It carries a central load of 1kN.It is supported in bearings, the distance between their centres being 1m.Determine the diameter of shaft assuming minor shock load with permissible shear stress of 45N/mm2 and bending stress of 70N/mm2. | CO1 | 16 |
| 5. | a. | List the various types of keys with their merits and demerits. | CO1 | 4 |
|  | b. | Design the rigid flange coupling is to designed to transmit 20kW at 1000rpm,assuming suitable allowable stresses. | CO1 | 16 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss in brief about the classifications of pressure vessels. | CO1 | 4 |
|  | b. | A plate of 90mm wide and 15mm thickis welded on to another plate by a single transverse weld and a double parallel fillet weld.Find the length of the parallel fillet weld if the plate is loaded by a static tensile load.Take the allowable tensile tensile stress of the plates is 70N/mm2 and weld shear stress as 55 N/mm2. | CO1 | 16 |
| 7. |  | Design a CI piston for a single acting four stroke I.C.engine for the following specifications. Cylinder bore 100mm,stroke length120mm,maximum gas pressure6MPa,Brake mean effective pressure 0.7Mpa,Fuel consumption 0.24kg/kW/hr,Speed2200rpm | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Specify the materials by which the crankshafts are made off. | CO1 | 2 |
|  | b. | A multicylinder engine is to run at a constant load with a speed of 600rpm.On drawing the crank effort diagram to scale 1cm=2500N-m and 1cm=30° ,the area above and below the mean torque line are in cm2 as follows. +1.6.-1.72,+1.68,-1.91,+1.97,-1.62.The speed is to be kept within ±1%of the mean speed of the engine. Find the suitable dimensions of a CI flywheel rim assuming suitable proportions. | CO2 | 18 |
|  |  | Compulsory: |  |  |
| 9. |  | The connecting rod of a petrol engine is to be designed for the following data. Piston diameter 80mm, stroke 120mm, weight of reciprocating parts15N, length of connecting rod 240mm, maximum speed 2800 rpm, explosion pressure corresponding to 10° of crank angle is 3MPa, factor of safety 6. If the connecting rod is to be made of 40Cr1 steel, find the dimensions of the I-section connecting rod. | CO2 | 20 |