Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 14ME2046 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | METAL CUTTING THEORY AND PRACTICE. | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate between orthogonal and oblique cutting. | CO1 | 5 |
|  | b. | During a metal cutting process under orthogonal cutting, a cutting force of 1000 N and feed force of 1020 N is observed when cutting with a speed of 200 m/min. The rake angle of the tool is 10 degrees and the shear angle is found to be 20 degrees. With the help of Merchant circle diagram, show the different forces and find their values. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. |  | Derive and explain with the following theories to determine the shear angle in orthogonal metal cutting. - Merchant and Ernst Theory. - Lee and Shaffer Theory. | CO1 | 20 |
| 3. | a. | List the different types of chips produced during machining operations. Explain the conditions leading to its generations with neat sketches. | CO2 | 10 |
|  | b. | What are the various international standards used to represent a cutting tool? Explain the importance of various angles used in these representations. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | What are the different types of systems used to represent the nomenclature of a single point cutting tool? Explain the angles and parts of the cutting tool with any one of the systems. | CO2 | 20 |
| 5. | a. | Describe the sources of heat generation during a machining process. | CO3 | 5 |
|  | b. | Classify the different type of coolants employed in machining process. Explain the application of each classification. | CO3 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | List the type of coatings applied to inserts. Explain how these coatings improve the performance of the cutting. | CO5 | 15 |
|  | b | How are index able inserts represented by ISO code? | CO2 | 5 |
| 7. | a. | State the reasons for tool wear. | CO5 | 5 |
|  | b | Explain the wear mechanisms associated with - Crater wear. - Flank wear. | CO5 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Define machinability index. | CO5 | 2 |
|  | b. | Enumerate the factors affecting tool life. Explain the effect of each factor on tool life. | CO5 | 8 |
|  | c. | State the basic objectives of efficient and economic machining. | CO5 | 5 |
|  | d. | A tool life of 80 minute is obtained at a speed of 30 mpm and 8 minute at 60 mpm. Determine the Tool life equation. | CO5 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define Chatter. List the effects of chatter. | CO4 | 5 |
|  | b. | List the different types of machine tool vibrations and explain the causes. | CO4 | 10 |
|  | c. | Suggest steps to avoid chatter in machining. | CO4 | 5 |