Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code | 14ME3028 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name | ADVANCED MECHANICAL VIBRATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | A machine part of mass 2kg vibrates in a viscous medium. Determine the damping coefficient when a harmonic exciting force of 25N results in a resonant amplitude of 12.5 mm with a period of 0.2 second. If the system is excited by a harmonic force of frequency 4HZ, what will be the percentage increase in the amplitude of vibration when the damper is removed as compared with that with damping? | CO4 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | An aircraft radio transmitter weighs 26kg and is mounted on five springs which deflect 8mm when the transmitter is placed upon them. Neglecting damping, find the percentage of engine vibration received by the transmitter for engine speeds of 1500 rpm and 2500 rpm. | CO4 | 10 |
|  | b. | A vibrating system consists of a mass of 50 kg, a spring of stiffness 30 x 103 N/m and a damper. The damping provided is only 20% of the critical value. Determine the damping factor, critical damping co-efficient, natural frequency of damped vibration, logarithmic decrement. | CO4 | 10 |
| 3. |  | A 300kg machine is placed at the end of a cantilever beam of length 1.8m, Elastic modulus 200GPa, and Moment of Inertia 1.8x10-5 m4. When the machine operates at 1000 rpm, it has steady state amplitude of 0.8mm. What is the steady state amplitude of the machine when a 30kg absorber of damping coefficient 650Ns/m and stiffness 1.5x105 N/m is added to the end of the beam? | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A diesel engine of weight 3000N is supported on a pedestal mount. It has been observed that the engine induces vibration into the surrounding area through its pedestal mount at an operating speed of 6000rpm. Determine the parameters of the vibration absorber that will reduce the vibration when mounted on the pedestal. The magnitude of the exciting force is 250N and the amplitude of motion of the auxiliary mass is to be limited to 2mm. | CO3 | 20 |
| 5. |  | Derive the relation between Dynamic and Static amplitude of motion for Forced Vibration of Undamped systems. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | A refrigerator unit of mass 40kg is to be supported by three springs each of stiffness k. The unit operates at 520 rpm. Find the value of stiffness if only 12% of the shaking force is allowed to be transmitted to the supporting structure. | CO4 | 10 |
|  | b. | With example, explain semi-definite system. | CO1 | 10 |

| 7. |  | Use matrix iteration method to determine the natural frequencies of the system shown in figure. X1 X2 X3 4m 2m m 3K K K | CO2 | 20 |
| --- | --- | --- | --- | --- |
| (OR) |  |  |  |  |
| 8. |  | Using Holzer’s method, find the natural frequency of the system shown in figure. Assume m1=m2=m3=1Kg & K1=K2=K3=1N/m. k1 k2 k3 m1 m2 m x1(t) x2(t) m3 m x3(t) | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | With neat sketch, explain the working of vibration generation equipment. | CO3 | 20 |

ALL THE BEST