Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Question Paper – Apr/May – 2017

| Code : | 14ME3040 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ENGINEERING FRACTURE MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div. | Questions | Course Outcome |  | Marks |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | a. | Discuss the Griffith failure and analysis for safe crack using strain energy release rate. | CO3 |  | 12 |  |
|  | b. | Differenciate intergranular and transgranular fracture. And state why swords offer more resistance to fracture. | CO2 |  | 8 |  |
| (OR) |  |  |  |  |  |  |
| 2. | a. | What are the modes of fracture? and explain the mechanism behind those. | CO2 |  | 12 |  |
|  | b. | Discuss the characteristics of ductile fracture and brittle fracture. | CO2 |  | 8 |  |
| 3. | a. | Why R curve is a straight line for brittle material? And how it is varies for ductile material? | CO2 |  | 12 |  |
|  | b. | Why the shape of the plastic zone is determined approximately. Mention the methods used for its computation. | CO2 |  | 8 |  |
| (OR) |  |  |  |  |  |  |
| 4. | a. | What is safe crack? And what is the role of safe crack in stability and unstability of fracture? | CO1 |  | 13 |  |
|  | b. | The fracture resistance is decreasing when the material is subjected to lower temperature. State the reason. | CO1 |  | 7 |  |
| 5. | a. | Derive Irwin approach for the correction of plastic zone to accommodate stresses in yield zone. | CO3 |  | 13 |  |
|  | b. | Explain why the crack tip plays vital role in fracture mechanics. | CO1 |  | 7 |  |
| (OR) |  |  |  |  |  |  |
| 6. |  | Explain J integral and drive the expression of J integral for beam with usefulness. | CO3 |  | 20 |  |
| 7. |  | An bar of steel alloy used to conduct a J integral test. The specimen had dimensions of B = 10 mm, W = 20 mm, S = 80 mm and a = 10 mm. The alloy possessed the following mechanical properties: E = 20 GPa and μ = 0.25. The area of the curve = 5 Nm2 and the maximum load = 15 kN. Calculate the value of J for this material. | CO3 | 20 |  |  |
| (OR) |  |  |  |  |  |  |
| 8. |  | Which mechanism is slowing the progressing of dislocations? Explicate the types of dislocation. | CO1 |  | 20 |  |
|  |  | Compulsory: |  |  |  |  |
| 9. |  | List the types defects can be identified using non destructive techniques. And discuss the common non destructive techniques. | CO1 |  | 20 |  |

ALL THE BEST