Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 15AE3009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FINITE ELEMENT ANALYSIS IN AEROSPACE APPLICATION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. | Write the engineering applications of FEM. | CO1 |  | 10 |
|  | b. | Write the engineering advantages of FEM. | CO1 |  | 5 |
|  | c. | Write the properties of the Stiffness matrix. | CO1 |  | 5 |
| (OR) |  |  |  |  |  |
| 2. |  | Consider a bar as shown in figure. An axial load of 200 kN is applied at point P. Take A1 = 2400 mm2, E1 = 70 x 109 N/m2, A2 = 600 mm2, E2 = 200 x 109 N/m2. Calculate the following - The nodal displacement at point P. - Stress in each material. - Reaction force. | CO1 |  | 20 |
| 3. |  | Derive the Shape function and Element Stiffness Matrix for Triangular element in global coordinate system. | CO1 |  | 20 |
| (OR) |  |  |  |  |  |
| 4. | a. | Write the properties of the Shape function. | CO1 |  | 5 |
|  | b. | Derive the shape function of eight node quadratic quadrilateral element in natural coordinate system. | CO2 |  | 15 |
| 5. |  | Derive the shape function of one dimensional linear quadratic element in global coordinate system. | CO1 |  | 20 |
| (OR) |  |  |  |  |  |
| 6. |  | For the two bar truss shown in figure, determine the displacements of node 1 and stress in element 1-3. |  | CO2 | 20 |
| 7. |  | Determine the stiffness matrix for the constant strain triangle (CST) element shown in figure. The coordinates are given in millimeters. Assume plane stress conditions. Take E = 210 GPa, v = 0.25 and t = 10 mm. |  | CO2 | 20 |
| (OR) |  |  |  |  |  |
| 8. |  | For the plane stress element shown in figure, the nodal displacements are U1 = 2.0 mm v1 = 1.0 mm U2 = 0.5 mm v2 = 0.0 mm U3 = 3.0 mm v3 = 1.0 mm Determine the element stresses σx,σy, τxy, σ1 and σ2 and the principal angle θP. Let E= 210 GPa, ν = 0.25 and t = 10 mm. All co-ordinates are in millimeters. | CO2 |  | 20 |
|  |  | Compulsory: |  |  |  |
| 9. |  | For the two dimensional sandy soil region shown in figure, determine the potential distribution and velocity gradient. The potential (fluid head) on the left side is a constant 10.0 m, and that on the right side is 0.0 m. The upper and lower edges are impermeable. The permeabilities are kx = ky = 25 x 10 – 5 m/s. Assume unit thickness. | CO2 |  | 20 |

ALL THE BEST