Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 15CE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | IRRIGATION ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the factors that govern the necessity of irrigation. | CO1 | 10 |
|  | b. | Briefly explain the types of irrigation with neat sketches. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Enumerate the methods of improving duty? | CO1 | 10 |
|  | b. | An irrigation canal has gross commanded area of 80,000 hectares out of which 85% is culturable irrigable. The intensity of irrigation for Kharif season is 30% and for Rabi season is 60%. Find the discharge required at the head of canal if the duty at its head is 800 hectares/cumec for Kharif season and 1700 hectares/cumec for Rabi season. | CO1 | 10 |
| 3. | a. | A stream of 135 litres per second was diverted from a canal and 100 litres per second were delivered to the field. An area of 1.6 hectares was irrigated in 8 hours. The effective depth of root zone was 1.8 m. the runoff loss in the field was 432 cu.m. The depth of water penetration varied linearly from 1.8 m at the head end of the field to 1.2 m at the tail end. Available moisture holding capacity of the soil is 20 cm per meter depth of soil. Determine the water conveyance efficiency, water application efficiency, water storage efficiency and water distribution efficiency. Irrigation was started at a moisture extraction level of 50 percent of the available moisture. | CO3 | 15 |
|  | b. | List out the factors that affect evapo transpiration. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Define rain gauge. What are the different raingauges used ? Explain them with sketches. | CO1 | 15 |
|  | b. | Write a note on unit hydrograph. | CO2 | 5 |
| 5. | a. | Define the terms i) Aquifer, ii) Aquiclude iii) Aquifuge iv) Aquitard v) specific yield. | CO2 | 10 |
|  | b. | Explain the steady flow towards a well in confined aquifer. Derive the equation. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Briefly explain about the classification of canals. | CO2 | 10 |
|  | b. | Write short notes on i) Yield of an open well ii) well losses. | CO3 | 10 |
| 7. |  | Design a regime channel of or a discharge of 50 cumecs and silt factor 1:1 using Lacey's Theory. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Define non-aluvial channel. Elaborately describe the method of design and procedure adopted for non-alluvial channel. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss the necessity of river training works.s Describe in brief different types of river training works? | CO3 | 12 |
|  | b. | Write a note on salinity and waterlogging and the method to control it. | CO3 | 8 |

ALL THE BEST