Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 15PH3010 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTROMAGNETIC THEORY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div | Questions | Course Out come | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Express the condition for Gauss’s law and prove it. Derive the expression for D due to the infinite sheet of charge placed in z=0 plane, due to point charge, line charge using Gauss’s law. Starting from the Gauss’s law as applied to the differential volume element, explain the concept of divergence. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Obtain an expression for total force experienced by a point charge due to infinite number of point charges around it. | CO1 | 12 |
|  | b. | Explain the procedure of obtaining E due to the line charge, surface charge and volume charge. | CO1 | 8 |
| 3. | a. | What is a dipole? Derive an expressions for potential and electric field intensity due to a dipole using method of images. | CO1 | 15 |
|  | b. | Four point charges of 10µc each are placed at the corners of square of side 1 m. Determine the value of the charge that is to be placed at the centre of the square so that this system of charges is brought to equilibrium. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Deduce the expressions for the electric field intensity of a point charge in the presence of grounded conducting sphere. | CO1 | 15 |
|  | b. | A point charge of 10 µc is located at (1, 2, 3) and another point charge of -3 µc is located at (3, 0, 2) in vacuum. Find the force between them. | CO2 | 5 |
| 5. |  | State and explain Biot-Savart law for the magnetic field B. Obtain an expression for the magnetic induction at a distance‘d’ from an infinitely long straight wire in which flows a current I. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Obtain an expression for energy stored in the magnetic field and also derive the expression for magnetic energy density | CO1 | 20 |
| 7. | a. | Derive the Maxwell’s equations in both differential forms and integral forms. | CO1 | 16 |
|  | b. | Explain the significance of displacement current . Write the Maxwell’s equations in which it is used. | CO1 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the relations for Poynting theorem. | CO1 | 12 |
|  | b. | A medium has constant conductivity of 0.1 mho/m, µr = 1 εr = 30. When these parameters do not change with the frequency, interpret whether the medium behaves like a conductor or a dielectric at 50kHz and 10GHz. | CO2 | 4 |
|  | c. | Given εr = 10, σ = 5 x 10-3 mho/m, µr = 1, evaluate the conducting behavior of ground at 1kHz, 10 MHz and 10 GHz. | CO2 | 4 |
|  |  | Compulsory: |  |  |
| 9. |  | Derive an expression for an intrinsic impedance, propagation constant and velocity of a plane electromagnetic wave when propagated in a lossy dielectric medium, conducting medium and good dielectric medium. | CO1 | 20 |