Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 16AE2001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | STRUCTURAL MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | An axial pull of 35000N is acting on a bar consisting of three lengths as shown in ig. below.If the Young’s Modulus is 2.1×105 N/mm2,determine: - Stresses in each section - Total elongation of the bar | CO1 | 10 10 |
| (OR) |  |  |  |  |
| 2. |  | A brass bar having cross-sectional area of 1000mm2 , is subjected to axial forces as shown in fig. below. Find the total elongation of the bar. Take E = 1.05 x105 N/mm2. | CO1 | 20 |
| 3. |  | Derive the expression for bending stress in terms of bending moment, moment of inertia and radius of curvature with suitable assumptions. | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Derive the expression for slope and deflection of a Simply supported beam carrying a Uniformly distributed load using Moment Area method. | CO4 | 20 |
| 5. |  | Two shafts of the same material and of same lengths are subjected to the same torque,if the first shaft is of a solid circular section and the second shaft is of hollow circular section,whose internal diameter is 2/3rd of the outside diameter and the maximum shear stress developed in each shaft is the same.Compare the weights of the shaft. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | The tensile stresses at a point across two mutually perpendicular planes are 120 N/mm2 and 60 N/mm2. Determine the normal,tangential and resultant stresses on a plane inclined at 30˚ to the axis of minor stress using Mohr’s circle method. | CO1 | 20 |
| 7. |  | A copper flat measuring 60mm×30mm is brazed to another 60mm×60mm mild steel flat as shown in ffig. Below.If the combination is heated through 120˚C, determine the following: - The Stress produced in each of the bar - Shear force which tends to rupture the brazing - Shear stress | CO1 | 10 5 5 |
| (OR) |  |  |  |  |
| 8. |  | The Principal stress at a point in an elastic material are 200 N/mm2(tensile),100 N/mm2(tensile) and 50 N/mm2 (compressive). If the stress at elastic limit in simple tension is 200 N/mm2 , determine whether the failure of the material occur according to maximum principal strain energy and maximum shear stress theory.Take poisson’s ratio=0.3. | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | A Cantilever 1.5m is loaded with a uniformly distributed load of 2kN/m run over a length 125m from the free end.It also carries a point load of 3kN at a distance of 0.25m from the free end. Draw the shear force and bending moment diagram for the cantilever. | CO2 | 20 |

ALL THE BEST