Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2017

| Code : | 16EE1001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRICITY FOR ENGINEERS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | With necessary layout diagram, explain the working of all the components in Thermal Power Plant. | CO1 | 14 |
|  | b. | Determine the equivalent resistance between the terminals A and B in the circuit shown below and also calculate the total current if the source is 30V. B A | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | With the help of block diagram, explain the working of Standalone Solar PV power plant. | CO1 | 14 |
|  | b. | State Kirchoff’s Laws. | CO1 | 6 |
| 3. | a. | Draw the one line diagram of a typical power supply system. Mark all the components. | CO1 | 6 |
|  | b. | Calculate the monthly electricity bill for a home with the following load specifications. S.No; Name of the Load; Quantity; Wattage; Operating Hours. 1; Fluorescent lamp; 4; 40W; 6. 2; Ceiling Fan; 3; 60 W; 12. 3; Refrigerator (165 L); 1; 100 W; 24. 4; Air Conditioner; 1; 1500 W; 6. 5; Mixer; 1; 450 W; 1. 6; LED Television; 1; 100 W; 7. Assume the following EB tariff. Category; Units consumed; Cost/unit (Rs/KWh); Fixed Charges. Consumption above 500 units; 0 – 100 units; 0; Rs.50/service. 101 – 200 units; 3.50. 201 – 500 units; 4.60. Above 500 units; 6.60. | CO1 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Draw the wiring circuit of fluorescent lamp and stair case wiring and explain its working. | CO1 | 12 |
|  | b. | Compare overhead transmission line and underground cable system. | CO1 | 8 |
| 5. | a. | Discuss the operation of offline Uninterrupted Power supply with necessary diagrams. | CO1 | 10 |
|  | b. | List down the applications of Electric Heating. | CO1 | 5 |
|  | c. | Discuss the steps to convert existing home into smart home. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | List down the electrical hazards with the effects of electricity on human body and also discuss the electrical protective devices. | CO1 | 12 |
|  | b. | State the Faraday’s law of electromagnetic induction and explain the types of induced emf with neat diagrams. | CO2 | 8 |
| 7. | a. | Sketch the constructional diagram of a three phase induction motor and explain its operation. | CO2 | 10 |
|  | b. | Identify the reason(s) for the single phase induction being not self staring. List down the methods to overcome and explain in detail any one method with neat diagram. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Elucidate the construction and working of an Electric Vehicle with a neat sketch. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Construct a system that will automatically control the water level in overhead tanks. | CO3 | 8 |
|  | b. | Develop a system for irrigation which will perform its operation without manual interruption. | CO3 | 12 |

ALL THE BEST