Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AIRCRAFT STRUCTURES								       Time: 3 hours

Subject Code: 09AE202				      				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	State the units of shear flow.

2.	Define elastic axis.

3.	Define load factor for an aircraft.

4.	What is the function of spars in a wing structure?

5.	Name one analytical method which is used to analyze trusses.

6.	A continuous beam is a statically indeterminate structure. Say True/False.

7.	Define Resilience.

8.	Give the expression for strain energy due to bending.

9.	Define buckling factor.

10.	Give the value for effective length of a column when both ends are hinged.

PART – B (5 x 3 = 15 MARKS)

11.	Show that torque T = 2qAm.

12.	What is a shear resistant web?

13.	Explain the method of joints for a truss.

14.	Explain Maxwell’s reciprocal theorem.

15.	Differentiate between a beam and a column.

PART – C (5 x 15 = 75 MARKS)

16.	An equal angle section is subjected to a shear force P acting along the direction of Y – axis, which is an axis of symmetry. Each leg of the angle has a centerline length b and thickness t. Derive the formula for the maximum shear stress in the angle. Draw the shear stress distribution.

(OR)

17.	Find the shear flow and twist per unit length of a two cell structure as shown in the figure below. Take G=25x105 N/cm2 and uniform thickness t=0.1cm. Torque applied T = 10,000Ncm.

18.	Discuss in detail about the wing structural analysis and fuselage stress analysis.

(OR)

19.	Explain the analysis of a complete tension field beam with a neat sketch.

[P.T.O]

20.	A beam supported at the supports A and C and continuous over the support B. Assuming EI is constant draw the bending moment and shear force diagram.

(OR)

21.	A continuous beam ABCD, shown in the figure below, having three equal spans of length “l” each carries a uniformly distributed load of w/unit length over its entire length. It is freely supported on all supports. Draw the SFD and BMD.

22.	A circular bar of dia “d” is bent into a shape of U of radius “R” and a straight portion of length l.  Equal loads P are applied at ends A and E. Determine the relative shift between A and E.

(OR)

23.	Determine the vertical deflection at point B for a cantilever beam shown in the figure below using unit load method.

24.	Derive the Euler’s buckling formula for a column when:

a.	both ends are fixed 		b.	one end is fixed and other end is free.

(OR)

25.	A shaft is subjected to a maximum torque of 10KNm and a maximum moment of 7.5 KNm at a particular section. If the allowable equivalent stress in simple tension is 160 MN/m2 ,find the diameter of the shaft according to the maximum shear stress theory.