Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AERODYNAMICS  										       Time: 3 hours

Subject Code: 09AE205							               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	An air foil is a streamlined object capable of producing more lift than drag force in a uniform stream of air. True or False?

2.	Cp = (p−p∞)(12ρ V∞2)and CD =D 12ρ V∞2S. Say True or False?

p – pressure; V- velocity; S – Surface Area; C – coefficient, are proper non – dimensional parameters.

3.	A flow is said to be irrotational if the curl of the velocity vector is zero. True or False?

4.	A source flow and sink flow in a plane look alike except for the direction of flow and volume rate of flow. True or False?

5.	Any object moves forward due to thrust force (a forward force) acting on it. True or False?

6.	Reciprocating type Aerospace Engines are very efficient and economical at lower attitudes. 	True or False?

7.	Maximum distance travelled by an airplane of passenger type could be different from range covered by the airplane. True or False?

8.	Maximum speed of an airplane can be different from cruising speed of an airplane. True or 	False?

9.	For aerospace vehicles, forces and moments measurements are more economically carried out in a short time using wind tunnel balances of proper ranges than any other methods. True or False?

10.	Models that are tested in wind tunnels can be either scaled – up or scaled – down or, correct scale models of the object of interest. True or False?

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between streamlined and bluff objects.

12. 	Describe a source flow in a plane.

13.	What are the types of propellers useful to airplanes?

14.	What is steady, straight and level flight of an airplane?

15. 	What is GEOMETRIC SIMILARITY requirement of a model tested in a WIND TUNNEL?

PART – C (5 x 15 = 75 MARKS)

16. 	Explain in detail the wing and airfoil section geometry.

(OR)

17. 	a.	Explain the forces and moments acting on an airplane.							(8)

b.	Sketch the pressure distribution on an airfoil at different .  is the angle of attack.	(7)

18.	A source and a sink of strength 4 m2/s and 8 m2/s are located at (–1,0) and (1,0) respectively.  Determine the velocity and stream function at a point P(1,1) which is lying on the flownet of the resultant streamline.

(OR)

[P.T.O]

19. 	A uniform flow with a velocity of 3 m/s is flowing over a plane source of strength 30 m2/s.  The uniform flow and source flow are in the same plane.  A point P is situated in the flow field.  The distance of the point P from the source is 0.5 m and it is at an angle of 30º to the uniform flow.  Determine 		a.	Stream function at point P 		b.	resultant velocity of flow at P

c.	location of stagnation point from the source.

20. 	Write short notes on:

a.	Drag of airplane components. 													(5)

b.	Total Drag Estimation.														(5)

c.	methods of Drag Reduction.													(5)

(OR)

21. 	Explain in detail about Aerospace Engines.

22. 	An aeroplane weighs 1,60,000 N and has a wing area of 42 m2.  The aeroplane engines provide a net thrust of 27000 N and the Drag Equation of the aeroplane is CD = 0.014 +0.05CL2

a.	At sea level, calculate the rate of climb for small angles of climb at a flight speed of 100 m/s.

b.	Calculate the maximum rate of climb at small angles of climb.

(OR)

23. 	Write short notes on:

a.	Cruise climb.																	(5)

b.	Straight and level flight range.													(5)

c.	Straight and level flight endurance.												(5)

24. 	 Write short notes on:

a.	Non – dimensional parameters.													(3)

b.	Geometric similarity 															(3)

c.	Kinematic similarity.															(3)

d.	Dynamic similarity.															(3)

e.	Model mounting in Wind Tunnels.												(3)

(OR)

25. 	Write short notes on:

a.	Streamline.																	(3)

b.	Pathline.																		(3)

c.	Streakline.																	(3)

d.	Pitot and Pitot –Static probe for pressure measurements.							(3)

e.	Flow visualization in wind and water tunnels.									(3)