Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: GAS DYNAMICS										       	       Time: 3 hours

Subject Code: 09AE207							               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	A one dimensional inviscid  flow can be a steady or unsteady flow (Yes / No).

2.	In general any stable fluid flow must satisfy conservation of Mass, Momentum and Energy of fluid (Yes / No).

3.	A normal shock wave is a shock wave bearing 90º with the fluid stream (Yes / No).

4.	A shock wave need not be attached to an object which creates that shock (Yes / No).

5.	A small perturbation potential theory means a potential flow with all the essential flow field parameters vary very little from the undisturbed free stream parameters (Yes / No).

6.	Prandtl – Glauert affine transformation is a transformation which transforms a compressible flow domain into an incompressible flow domain in a modified co-ordinate system for velocity potential function (Yes / No).

7.	A High speed airfoil in a High speed fluid flow should yield more lift force than the drag force within the specified angles of attack (Yes/No).

8.	A low subsonic airfoil configuration is completely different from high supersonic airfoil configuration (Yes / No).

9.	A High speed blow down tunnel can be either a continuous or intermittent type (Yes / No).

10.	In an induction tunnel, the ambient atmospheric air flows through the test section and the inducing high pressure air does not flow through the test Section (Yes / No).

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between velocity of fluid and velocity of sound.

12. 	What is a normal shock wave? What are the other kinds of shock waves available?

13.	Distinguish between Mach wave and Shock wave in a compressible flow.

14.	Draw a few subsonic and high speed airfoils.

15. 	What are the flow visualization techniques available for low speed and high speed flows?

PART – C (5 x 15 = 75 MARKS)

16. 	Derive the generalized continuity equation for a three dimensional, viscous and compressible fluid flow in a compressible flow domain.

(OR)

17. 	Explain the performance of a convergent-divergent flow passage under various back pressures.

18. 	 A blast wave created by a nuclear explosive, waves with a velocity of 15,240 m/sec into still atmosphere air having a pressure of 101335 N/m2 and a temperature of 294.4 K. Calculate

a.	the Mach number of the shock wave relative to the stationary air.

b.	the stagnation values of the pressure and the temperature behind the shock wave relative to 	a stationary observer.

(OR)

[P.T.O]

19. 	Air approaches a symmetrical wedge θ = 15º at Mach number 2.  Determine for the strong and weak waves:

a.	Wave angle.						b.	Pressure ratio.

c.	Density ratio. 					d.	Downstream Mach number.

20. 	a.	Obtain an expression for Prandtl-Glauert affine transformation for subsonic flows.	(12)

b.	Differentiate Mach wave and Mach angles.										(3)

(OR)

21. 	From the fundamental equations of fluid flow, derive small perturbation potential flow equation.

22. 	Bring out expressions for mass rate of flow through a convergent – divergent nozzle and area ratio of nozzle.  Also show that for each Area ratio, two values of Mach number are possible.

(OR)

23. 	Write short notes on:																(5x3)

a.	Critical Mach number of airfoil.

b.	Upper critical Mach number.

c.	Lower critical Mach number.

d.	Drag – Divergence Mach number.

e.	Transonic Area rule for a flying object.

24. 	 Present neatly the layouts of High speed wind tunnels.

(OR)

25. 	a.	How do you use pitot static tube available for subsonic wind – tunnel measurement in 	compressible and supersonic flows?											(8)

b.	Explain any one optical method for flow visualization.							(7)