Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SPACE DYNAMICS									      	       Time: 3 hours

Subject Code: 09AE208			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define the specific impulse and mass ratio of a rocket.

2.	Give the thrust equation of a rocket motor.

3.	What is the need for stability and control for rockets?

4.	Differentiate between stability and control.

5.	What is a satellite?

6.	What are Asteroids?

7.	What is injection epoch?

8.	What is Hohmann transfer trajectory?

9.	Give a note on the Orbiter mission.

10.	What is the energy necessary to escape from the earth?

PART – B (5 x 3 = 15 MARKS)

11.	Discuss about the separation of stages.

12.	Differentiate between static and dynamic stability.

13.	Can gravity affect the surface of objects in orbit around each other?

14.	Calculate the nodal regression for a satellite in a 150 km Earth orbit of eccentricity 0.2 and inclined at 63.4 deg to the equator. (Radius of the Earth is assumed to be 6378 km).

15.	Calculate the velocity of a space vehicle with respect to the Sun at a low earth orbit launch point for a Mars landing mission. Assume that the orbit of the Earth around the Sun is circular.

(Mean distance from the Earth to the Sun is 1 AU which is 149.6 x 106 km, mean distance from the Mars to the Sun is 1.524 AU, Sun’s gravitational parameter µs = 1.327 x 1011 km³/s², Earth’s gravitational parameter, µe = 398600.4418 km³/s²)	.

PART – C (5 x 15 = 75 MARKS)

16. 	a.	A two-stage rocket has the following masses: 1st-stage propellant mass 120,000 kg, 1st-		stage dry mass 9,000 kg, 2nd-stage propellant mass 30,000 kg, 2nd-stage dry mass 3,000 	kg and payload mass 3,000 kg. The specific impulses of the 1st and 2nd stages are 260s 	and 320s respectively. Calculate the rocket's total ∆V.								(10)

b.	Explain about the staging of rockets.			            		  					(5)

(OR)

17.	Mention the stage separation techniques for a multistage launch vehicle in atmosphere. Describe any two techniques with neat illustrations.

18.	a.	Discuss about the static longitudinal stability and control of rockets.				(10)

b.	Explain the phenomena of dynamic stability.										(5)

(OR)

19.  	Explain in detail about the lateral and directional stability and control of rockets.

[P.T.O]

20.	a.	Describe in detail about the Solar system and Earth’s atmosphere.					(10)

b.	Calculate the apsidal rotation rate for a satellite in a 400 km circular orbit at i = 60 deg 			inclination to the equator. (Radius of the earth is assumed to be 6378 km).                	(5)

(OR)

21.	a.	Explain the effect of space environment on the selection of materials of spacecraft.	(8)

b.	Why is it better to launch a spaceship from near the equator?						(7)

22.  a.	Explain the method of vibrations of orbital elements. 					 			(8)

b.	Explain in detail the orbit deviations due to injection errors.						(7)

(OR)

23.	a.	Estimate the total ΔV requirement for a Hohmann transfer from a 150 km altitude of 	circular parking orbit around the earth to a 150 km altitude of circular capture orbit around 	the planet Mercury. Furthermore, assume that the planets have coplanar circular orbits. 	(Mean distance from the Earth to the Sun is 1 AU which is 149.6 x 106 km, mean distance 	from the Mercury to the Sun is 0.387 AU, Sun’s gravitational parameter µs = 1.327 x 1011 	km³/s², Earth’s gravitational parameter, µe = 398600.4418 km³/s², Mercury’s gravitational 	parameter µm = 22032 km³/s² and the Radius of the Mercury is 2440 km).			(8)

b.	Explain in detail the Cowell’s Method and Encke’s Method.				   		(7)

24.	A satellite is launched from a circular equatorial parking orbit at an altitude of 160 km into a 	circular synchronous orbit by using a Hohmann transfer ellipse. Assume a homogeneous 	spherical earth with a radius of 6374 km. Determine the velocity increments for entering the 	transfer ellipse and for achieving the synchronous orbit at 42,200 km altitude.

(OR)

25.	a.	Describe the launch of Interplanetary Spacecraft.		 			 		 		(7)

b.	Discuss in detail the Two-dimensional Interplanetary Trajectories.			  		(8)