Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AIRCRAFT PROPULSION								       Time: 3 hours

Subject Code: 09AE209			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	An Engine working based on ___________ cycle produces 100% thermal efficiency.

2.	Name the different layers of an atmosphere.

3.	Define Slip factor.

4.	What is called surging of a compressor?

5.	List the types of combustion chambers.

6.	What are the advantages of a reaction turbine over impulse turbine?

7.	List few applications of thrust vectoring.

8.	What are the limitations of an after burner?

9.	What is called match point?

10.	Define operating line.

PART – B (5 x 3 = 15 MARKS)

11.	State the principle of a Jet engine. Give the classifications of Jet engines.

12.	What are the losses in a centrifugal compressor?

13.	List the factors affecting the combustion chamber design.

14.	State the functions of a Nozzle.

15.	What are the assumptions involved in determining the Operating line?

PART – C (5 x 15 = 75 MARKS)

16. 	What is by-pass-ratio? Explain the working principle of Turbofan engine with a neat diagram.

(OR)

17.	A gas turbine engine operates at a mass flow of 30 kg/s. Air enters the compressor at a pressure of 1 bar and temperature of 15˚C and is discharged from the compressor at a pressure of 10.5 bar. Combustion occurs at constant pressure and results in a temperature rise of 420K. If the flow leaves the turbine at a pressure of 1.2 bar, determine the net power output from the unit and also the thermal efficiency. Take Cp=1005 J/kg-K and γ=1.4.

18.	Explain the phenomenon of surging and choking of a Centrifugal compressor.

(OR)

19.	Air at a temperature of 290 K enters a ten stage axial flow compressor at a rate of 3kg/s. The pressure ratio is 6.5 and the isentropic efficiency is 90%, the compression process being adiabatic. The compressor has symmetrical blades. The axial velocity of 110m/s is uniform across the stage and the mean blade speed of each stage is 180 m/s.

Determine the direction of the air at entry to exit from the rotor and the stator blades and also the power given to the air. Assume Cp=1005J/kg-K and γ=1.4

20.	Discuss the different types of combustion chambers used in air craft engines with their relative merits and demerits. Draw neat sketches.

[P.T.O]

(OR)

21.	a.	Explain the methods used for turbine blade cooling.		                   				(7)

b.	Describe the working of an axial flow turbine stage with a neat sketch and draw the T-S diagram and velocity triangles. 				      								(8)

22.	What is called Thrust augmentation? Explain the methods used in aircraft for thrust augmentation.

(OR)

23.	Explain with neat sketches the different modes of operation of Supersonic inlets.

24.	Explain the matching procedure of a single spool turbo jet engine.

(OR)

25.	The following data refer to a single-shaft gas turbine operating at its design speed.

| Compressor characteristic |  |  | Turbine characteristic |  |
| --- | --- | --- | --- | --- |
| P02/P01 | mT01/P01 | ηc | mT03/P03 | ηt |
| 5.0 | 329 | 0.84 | 139.0 | 0.87 |
| 4.5 | 339 | 0.79 | 139.0 | 0.87 |
| 4.0 | 342 | 0.75 | 139.0 | 0.87 |

Assuming ambient conditions of 1.013 bar and 288 K, a mechanical efficiency of 98%, and neglecting all pressure losses, calculate the turbine inlet temperature required for a power output of 3800 kW.