Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED PROPULSION SYSTEMS					       Time: 3 hours

Subject Code: 09AE211			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	In _____________ turbine expansion of the gas takes place in both the stator and in the rotor.

2.	Define degree of reaction of a turbine.

3.	What do you mean by supercritical mode of operation of a Ramjet?

4.	The high expansion angle for a contour nozzle is _______.

5.	List the classifications of a nozzle.

6.	Name few solids used as a propellant in rockets.

7.	The cryogenic temperature of oxygen is ________.

8.	Define Specific Impulse.

9.	What are the applications of electric propulsion?

10.	What is called a ‘solar sail’?

PART – B (5 x 3 = 15 MARKS)

11.	List the differences between the impulse and reaction turbine.

12.	State the advantages and disadvantages of Air-augmented rockets.

13.	Explain the operations of aerospike nozzles.

14.	What are the basic requirements of a liquid propellant?

15.	What are the properties required for propellants used for nozzleless propulsion?

PART – C (5 x 15 = 75 MARKS)

16. 	The following particulars of a single stage turbine of free vortex type are given below:

Total head inlet pressure			: 4.6 bar

Total head inlet temperature		: 700˚C

Static head pressure at mean radius	: 1.6 bar

Mean blade diameter/blade height	: 10

Nozzle loss coefficient			: 0.10

Nozzle outlet angle				: 60˚

Determine the gas temperature, velocities and discharge angle at the blade root and tip radii. Take Cp=1.147 and γ=1.33 and mass flow rate = 20 kg/s.

(OR)

17.	Gas at 7 bar and 300˚C expands to 3 bar in an impulse turbine stage. The nozzle angle is 70˚ with reference to the exit direction. The rotor blades have equal inlet and outlet angles, and the stage operates with the optimum blade speed ratio. Assuming that the isentropic efficiency of the nozzle is 0.9, and that the velocity at entry to the stage is negligible, deduce the blade angle used and the mass flow required for this stage to produce 75kW. Take Cp=1150 J/kg-K

18.	With a neat diagram, explain the working principle of Ramjet. State its advantages and disadvantages, applications and also obtain an expression for ideal efficiency of a Ramjet.

(OR)

[P.T.O]

19.	Explain the sub-critical, critical and supercritical conditions with the help of sketches under which a Ramjet engine diffuser can operate.

20.	A rocket projectile has the following characteristics:

Initial mass							: 200 kg

Mass after rocket operation			: 130 kg

Pay load, structural masses			: 110 kg

Rocket endurance 					: 3 sec

Average specific impulse of propellant	: 240sec

Determine the vehicle’s mass ratio, propellant mass fraction, propellant flow rate, thrust, thrust-to-weight ratio, acceleration of vehicle, effective exhaust velocity, total impulse and impulse-to- weight ratio.

(OR)

21.	Design a nozzle for an ideal rocket that has to operate at 25 km altitude and gives 5000 N thrust at a chamber pressure of 2.068 MPa and chamber temperature of 2800 K. Assuming that K=1.30 and R=355.4 J/kg-K, determine the throat area, exit area, throat velocity, and exit temperature.

22.	Write notes on the following with respect to the solid propellant rocket motor: 
a.	Rocket motor case.						       									(7)
b.	Igniters.							       											(8)

(OR)

23.	Explain the salient features of a liquid propellant rocket motor with turbo-pump feed system with the help of a schematic diagram. Write down its advantages and
disadvantages.

24.	Explain the electric rocket and nuclear rocket propulsion techniques with neat sketches.

(OR)

25.	What is the concept involved in Ion propulsion? Explain with the help of a diagram how an Ion engine works.