Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AIRCRAFT PERFORMANCE							      	      Time: 3 hours

Subject Code: 09AE216						       	      		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	In a biplane aircraft _________________ drag is more compared to monoplane aircraft.

2.	The velocity of flow at the stagnation point is____________.

3.	How the drag divergence Mach number is increased by the wing sweep?

4.	Why most of the trainer aircrafts are having high aspect ratio wing?

5.	Why does propeller thrust decrease with altitude?

6.	At steady level flight conditions speed of the aircraft is zero. True / False?

7.	What is rate of climb?

8.	Name few thrust augmentation methods.

9.	What are the assumptions made in Froude’s momentum theory?

10.	A device which converts the translational energy of the fluid into rotational energy of the axis or vice versa is called as____________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is Magnus effect? List its applications.

12.	Compare cambered aerofoil and symmetrical aerofoil.

13.	Obtain the expression for velocity of stall and explain the factors affecting the velocity of stall.

14.	How are the leading-edge slots used as a high lift device?

15.	Draw the typical V-n diagram for an aircraft and mark the corner velocity and maneuvering 	point.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain Kutta-Joukowski equation for lift force produced by a rotating cylinder subjected to steady uniform flow.

(OR)

17.	a.	Differentiate between streamlined body and bluff body.			       				(5)

b.	Explain the NACA 6-digit and 7-digit aerofoil numbering systems.	     				(10)

18.	a.	An aircraft weighs 40,000 N, has a wing area of 350 m2 and a wing span of 50 m. At sea-	level the aircraft flies at a velocity of 200 and 600 m/sec in steady level. What are the 	values of the induced drag and the associated drag coefficients? Assume e= 0.85.	(8)

b.	Explain the importance of aspect ratio in aircraft design.		       					(7)

(OR)

19.	a.	Discuss the principle of boundary layer control method for skin friction drag reduction.(7)        b.	Explain the characteristics of a swept wing platforms. 			      				(8)

20.	An aircraft having wing area of 255 m2 and weight of  10000 kg and a clear drag polar (flaps and gear up) of CD=0.023+0.0735CL2. Calculate the following:

a.	(L/D) max

b.	VD min at sea level and at 40000 m

c.	T min for level flight

[P.T.O]

(OR)

21.	An airplane weighing 100000 N is powered by an engine producing 20000 N of thrust under sea level standard conditions. If the wing area be 25 m2, calculate:

a.	Stalling speeds at sea level and at 10 km altitude.

b.	(CD /CL) min, (CD/CL3/2) min, Trmin, Prmin, Vmd and Vmp under sea level conditions.

Assume CLmax = 1.5, CD = 0.016 + 0.064CL2.

22.	Define take off distance and derive a suitable formula for estimating the take of distance.

(OR)

23.	a.	A jet airplane has a weight of 922140 N and wing area of 158 m2. The weight of the fuel 	and oil together is 294300 N. The drag polar is given by:

CD = 0.017 + 0.0663 CL2

Obtain the maximum range in constant CL flight at an altitude of 10 km assuming the TSFC to be 0.95 hr-1.	                         											(7)

b.	Explain the thrust reversing mechanisms used in aircraft.		                   			(8)

24.	An airplane with a wing area of 20 m2 and a weight of 19,620 N dives with engine switched off, along a straight line inclined at 60o to the horizontal. What is the acceleration of the airplane when the flight speed is 250 kmph? If the airplane has to pull out of this dive at a radius of 200 m, what will be the lift coefficient required and the load factor? Drag polar is given by: CD = 0.035 + 0.076CL2 and the maneuver takes place around an altitude of 2 km.

(OR)

25.	What are the assumptions made in blade element theory? Explain blade element theory for propeller blades.