Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PRINCIPLES OF CHEMICAL ENGINEERING			       Time: 3 hours

Subject Code: 09BT203			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Ratio of gram moles of solute to mass of solvent in kg is defined as __________.

2.	For NTP conditions at 273.15K and 101.325kPa, volume occupied by 1kmol of gas is

a.	22.4 m3/sec. 		b.	20.4  m3/sec			c.	2.24 m3/sec

3.	A stream bled off to remove an accumulation of certain inerts or unwanted materials that might otherwise build up in recycle stream is called ________.

4.	In the reaction   A + B → C

C +D → D

the ratio of Moles of A reacted to produce C to moles of A totally reacted is known as

a. 	Yield     	b. 	Selectivity    c. 	Conversion  		d. 	Excess reactant

5.	Stoke (St) is the representation of _________ in cgs system.

6.	Pitot is an instrument used to measure ________.

7.	When fluid flows through a pipe line maximum velocity occurs at the walls of the pipe line (True /False).

8.	The wall thickness of a pipe is indicated by ________ number which increases with thickness.

9.	To avoid cavitation, the pressure at the pump inlet must exceed the vapor pressure by a certain value ________.

10.	Velocity head of the liquid from the impeller is converted into pressure head in a ________ pump

a.	Centrifugal		b.	Rotary  		c.	Plunger  		d.	Diaphragm.

PART – B  (5 x 3 = 15 MARKS)

11.	State the following laws:

a.	Dalton’s law				b.	Amagat’s law		c.	Raoult’s law

12.	Write the general and overall material balance with block diagram for:

a.	Distillation unit.			b.	Extraction unit

13.	Discuss in detail about the rhelogical behavior of non-newtonian fluids.

14.	Differentiate venturi meter and orifice meter.

15.	Explain the working principle of an Air lift pump.

PART – C  (5 x 15 = 75 MARKS)

16.	Make the following conversions

a.	294 g/l H2SO4 to normality		b.	4.8mg/ml CaCI2 to normality

c.	5 N H3PO4  to g/l					d.	54.75 g/l HCl to molarity

e.	3 M K2 SO4  to gm /lit

Data: Molecular weight of   H2SO4 =49, CaCI2 =55.5, H3PO4=49, K2 SO4 =174

(OR)

[P.T.O]

17.	A gas mixture has the following composition by volume:

SO2= 8.5%, O2=10% and N2=81.5%

Find	a.	Density of the gas mixture at a temperature at 473 K and 202.65kPa

b.	Composition by weight

Data: Molecular weight on SO2=64, O2=32, N2=28

18.	In the Deacon process for the manufacture of chlorine a dry mixture of hydrochloric 	acid gas 	and air is passed over a heated catalyst which promotes oxidation of acid. Air is used 30% in 	excess of that theoretically required kg of the acid. (Atomic weight of chlorine = 35.5,air 	contains 23.2% oxygen by weight).

Reaction : 4 HCL + O2  → 2Cl2 + 2H2O.

Calculate the following:

a.	The weight of air supplied per kg of acid.

b.	The composition (weight %) of gases entering.

c.	The composition (weight %) gases leaving, assuming that 60% of acid is oxidized in        			the process.

(OR)

19.	The producer gas made from the coke has the following composition by volume%: CO-28, 	CO2 – 3.5, O2 – 0.5, N2 -68. The gas is burnt with such a quantity of air that the oxygen from 	air is 20% in excess of the net oxygen required for complete combustion. If the combustion is 	98% complete calculate the weight of the gaseous product formed per 100kg of gas burnt.

20.	a.	Prove that pressure at any point is independent of direction.                             		(7)

b.	Derive the mathematical statement for the condition of hydrostatic equilibrium. 		(8)

(OR)

21.	a.	Show that in a laminar flow the velocity distribution with respect to radius is a           				parabola with the apex at the centerline of the pipe.                                       			(10)

b.	State Hagen Poiseuille equation and its application.                                         		(5)

22.	a.	Discuss in detail about the mechanism and application of fluidization.           		(7)

b.	Derive minimum fluidization velocity.                                                               	 	(8)

(OR)

23.	a.	Explain the working principle of area flow meter and its calibration.               		(10)

b.	Differentiate orifice and venturimeter.                                                              		(5)

24.	With a neat explain the construction, performance and working principle of centrifugal pump.

(OR)

25.	With a neat diagram explain the construction and working principle of positive displacement

reciprocating pump.