Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: UNIT OPERATIONS									       	       Time: 3 hours

Subject Code: 09BT205								              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Heat flow by conduction is governed by ______________law.

2.	The driving force for heat flow is _____________ difference.

3.	In the SI system the unit of overall heat transfer coefficient is _____________.

4.	The fraction of incident radiation absorbed is known as ______________.

5.	The shortest centre-to-centre distance between the adjacent tubes is known as the __________.

6.	______________ is carried out by supplying heat to a solution to vaporize a solvent.

7.	Filter aids are used to ______________ the filtration rate.

8.	Sedimentation means______________.

9.	Swirling and vortex are prevented using ______________.

10.	______________ mixer is used for dissolving rubber in liquid.

PART – B (5 x 3 = 15 MARKS)

11.	A wall of a furnace 0.224 m thick is constructed of a material having a thermal conductivity of 1.3 W/m K. This will be insulated on the outside with a material having an average k of 0.346 W/m K so that the heat loss from the furnace will be equal to or less than 1830 W/m2. The inner surface temperature is 1588 K and the outer 299 K. Calculate the thickness of insulation required.

12.	Explain the difference between drop-wise and film-wise condensation.

13.	Explain briefly about temperature patterns in multi pass exchangers.

14.	Describe the characteristics of a filter medium.

15.	Write short notes on mixing of gases with liquids.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the expression for heat conduction through a hollow sphere.					(7)

b.	Derive the expression for heat conduction through a hollow cylinder.				(8)

(OR)

17.	A steam boiler furnace is constructed out of a layer of fire clay 125 mm thick and a layer of red brick 500 mm thick. If the wall temperature inside the boiler furnace is 1373 K and on the outside wall is 323 K,

a.	Determine the amount of heat loss per m2 of the furnace wall.

b.	It is desired to reduce the thickness of the red brick layer in this furnace to half by filling in the space between the two layers by diatomite whose k=0.113+0.00016T, W/mK. Estimate the thickness of the filling to ensure an identical loss of heat for the same outside and inside temperature.

Data: k for fire clay = 0.533 W/mK and k for red brick = 0.7 W/mK. 	   	   (5+10=15)

18.	By using dimensional analysis derive the following relation for forced convection:

NNu = f(NRe, NPr)

(OR)

[P.T.O]

19.	a.	Dry steam at 373 K condenses on the outside surface of a horizontal pipe of 25 mm O.D. 	The pipe surface is maintained at 357 K by circulating water through it. Determine mean 	average heat transfer coefficient, heat transfer per unit length of pipe and condensate rate 	per unit length of pipe.

Data: The properties of condensate at the film temperature of 350 K are µ = 306 x 10-6 N.s/m2, k = 0.668 W/mK, ρ = 974 kg/m3, λ= 2225 KJ/kg.							(10)

b.	Calculate the rate of heat transfer by radiation from an unlagged steam pipe of  50 mm OD 		at  393 K to air at 293 K. Assume emissivity (e)=  0.9.							(5)

20.	Explain in detail about the construction details of a shell and tube heat exchanger with a 	diagram.

(OR)

21.	a.	Explain briefly about evaporator economy.										(5)

b.	Discuss in detail about the types of feeding in multiple effect evaporators.			(10)

22.	Describe in detail about the types of filtration processes.

(OR)

23.	Explain in detail about:

a.	Top suspended basket centrifuges.												(8)

b.	Batch sedimentation. 															(7)

24.	Write notes in detail about:

a.	Power for agitation.															(8)

b.	Mixing of viscous and plastic masses.											(7)

(OR)

25.	Elaborate on:

a.	Agitator scaleup.																(10)

b.	Emulsifiers.																	(5)