Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MASS TRANSFER OPERATIONS						       	       Time: 3 hours

Subject Code: 09BT206				       				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The ratio of diffusion coefficient in a gas to that in a liquid is in the order of:

a.	105		b.	10-5			c.	102			d.	10-2

2. 	According to film theory for equimolar counter diffusion, the mass transfer coefficient is given by ____________.

3. 	In a fractionating column minimum number of ideal stages is required when the reflux ratio is equal to ___________.

4. 	In the McCabe-Thiele diagram for binary distillation, vertical feed line represents	_____________ feed and horizontal feed line represents _____________ feed.

5. 	For absorption of sparingly soluble gas in a liquid:

a.	gas side mass transfer coefficient should be increased.

b.	liquid side mass transfer coefficient should be increased.

c.	liquid side mass transfer coefficient should be increased.

d.	mass transfer coefficient should be kept constant.

6. 	The ratio of liquid to gas flow rate in a counter current gas absorption column is increased at otherwise identical conditions. Which ONE of the following statements is TRUE?

a.	The operating line shifts towards the equilibrium line.

b.	The operating line shifts away from the equilibrium line.

c.	The operating line does not shift.

d.	 The concentrations of the absorbed species increases in the exit stream.

7. 	Define heat of wetting.

8. 	What is meant by adsorption hysteresis?

9. 	What is plait point?

10. 	What is a tie line?

PART – B (5 x 3 = 15 MARKS)

11. 	What is meant by Molecular diffusion and Eddy diffusion?

12. 	What is Flash distillation?

13. 	Define Absorption factor and Stripping factor.

14. 	Explain Langmuir and Freundlich isotherm.

15. 	Define Distribution coefficient and state its importance.

PART – C (5 x 15 = 75 MARKS)

16.	a. 	Ammonia gas is diffusing at a constant rate through a layer of stagnant air 1 mm thick. 	Conditions are such that the gas contains 50 % by volume ammonia at one boundary of the 	stagnant layer. The ammonia diffusing to the other boundary is quickly absorbed and the 	concentration is negligible at that plane. The temperature is 295 K and the pressure 	atmospheric, and under these conditions the diffusivity of ammonia in air is 1.8 x 10-5 	m2/s. Estimate the rate of diffusion of ammonia through the layer. 					(9)

b. 	Briefly explain the Two – film theory. 											(6)

[P.T.O]

(OR)

17. 	Briefly explain following:

a. 	Chilton-Colburn analogy between mass and momentum transfer in detail.			(8)

b. 	Steady state diffusion in multi component gaseous mixture.						(7)

18 .	a.	Write a short note on types of Distillation.										(5)

b. 	A liquid mixture containing 50 mol % n-heptane and 50 mol % n-octane at 30 0C is to be 	continuously flash vaporized at 1 atm to vaporize 60 mol % of the feed. Compute the 	composition of the composited distillate and residue. 								(10)

(OR)

19. 	A continuous fractionating column is required to separate a mixture containing 0.695 mole fraction n-heptane (C7H16) and 0.305 mole fraction n-octane (C8H18). n-heptane composition in distillate and residue are 99 and 1 mol % respectively. The feed is all liquid at its boiling-point, and this is supplied to the column at 1.25 kg/s. The boiling-point at the top of the column may be taken as 372 K, and the equilibrium data are:

| mole fraction of heptane in vapour | 0.96 | 0.91 | 0.83 | 0.74 | 0.65 | 0.50 | 0.37 | 0.24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mole fraction of heptane in liquid | 0.92 | 0.82 | 0.69 | 0.57 | 0.46 | 0.32 | 0.22 | 0.13 |

Determine the minimum reflux ratio required if the reflux used were twice the minimum possible. Also determine no of stages required.

20. 	An air-NH3 mixture containing 20 mole % NH3 is being treated with water in a packed tower to recover NH3. The incoming gas rate is 1000 kg h-1 m-2. The temperature is 35 0C and the total pressure is 1 atm. Using 1.5 times the minimum water flow rate, 95% of NH3 is absorbed. Hendry’s law is valid and ye = 0.746 x. Variation in gas flow rate may be neglected. Determine the number of transfer units required.

(OR)

21. 	Explain the following:

a.	Theories of gas absorption.													(8)

b.	Types of packing. 															(7)

22. 	a. 	Name any four commercial adsorbents with their industrial applications. 			(6)

b. 	Explain break through curve for fixed bed adsorption. 							(9)

(OR)

23. 	a. 	Write short notes on ion-exchange process. 										(10)

b. 	Compare physical adsorption and chemisorptions. 								(5)

24. 	An acetone-water mixture containing 20% acetone (by weight) is to be extracted at the rate of 1000 kg/hr using trichloroethane in a counter current extraction system. The water and trichloroethane are insoluble. If 90 % recovery of acetone is desired estimate the number of stages required. 1.5 times the minimum solvent is used. The equilibrium relationship is given by y = 1.65x where x and y are weight fractions of acetone in water and trichloroethane respectively.

(OR)

25. 	a.	What are the important factors that govern the selection of extractors? 				(5)

b.	Describe the construction and working of any two extraction equipments. 			(10)