Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DOWNSTREAM PROCESSING						 	      Time: 3 hours

Subject Code: 09BT222				       		        		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The three destabilizing factors for a bio product are:

a.	________ 	b.	_________	c.	________.

2.	The three physical methods of cell disruption are:

a.	________ 	b.	_________	c.	________.

3.	What is a filter aid and what is its use?

4.	What is the use of centrifugation?

5.	What is the main difference between adsorption and absorption?

6.	Define permeation.

7.	What is retention volume VR?

8.	What is the basis behind the use of gel filtration?

9.	The reason for polishing a biopharmaceutical is to achieve the________.

10.	What is freeze drying?

PART – B  (5 x 3 = 15 MARKS)

11.	With a suitable example define inter cellular and intra cellular products.

12.	How are centrifuges classified?

13.	Draw the line diagram for an aqueous two phase extraction process.

14.	List out the three chromatographic techniques.

15.	How will you distinguish between bound and unbound moisture?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the reason for having many downstream processes for bio products.

(OR)

17.	Describe the various methods of cell disruption in detail.

18.	A broth having 2 liters with a viscosity of 2.8 cp has 5.2% of compressible biomass. It was filtered on a filter of 10 cm dia at a pressure drop of 1.6 bar. The duration of filtration was 16 mins. The cake had a compressibility of 0.4. Calculate the time required to filter 2500 liters of the above said broth on a filter area of 2.5 sq.m at a pressure drop of 2.8 bar. Assume the resistance of the filter cake is negligible.											(2x7.5)

(OR)

19.	a.	Explain the operation of a disc bowl centrifuge.  									(7)

b.	A suspension of spherical particles of 0.1 mm diameter was allowed to settle in a column 	of 50 cm length. The density difference between the solid particles and the liquid was  	0.05 gms /cc.  The viscosity of the liquid was 1.1 cp.

i.	Calculate the settling time of the particles assuming that the particles reach their 	terminal velocity almost instantaneously.

ii.	Calculate the settling time in a centrifuge rotating at 400 rpm if the distance between			the axis of rotation and bottom of the centrifuge was 12 cm and the distance between 		the axis and the liquid surface was 3 cm.									(4+4)

[P.T.O]

20.	a.	Explain the basic principles of super critical fluid extraction with a neat sketch.		   (7)

b.	Enumerate the various advantages of the super critical fluid extraction.				(4+4)

(OR)

21.	a.	The partition coefficient of an organic acid in an organic solvent and water system is 2.7.

i.	Calculate the volume of organic solvent required to extract 99% of the acid from the       		50 ml of aqueous solution.

ii.	How many extractions with 50 ml of organic solvent would be required to extract 99 			% of the acid?															(4+3)

b.	Classify the various membrane separation processes and explain any one in detail.	(8)

22.	What are the basic principles of size exclusion chromatography? Describe in detail.

(OR)

23.	a.	Two proteins of molecular weights 2.5 x 105 and 1 x 104 were eluted out of a gel filtration 	column at 220 ml and 300 ml respectively. Determine the molecular weight of the protein  	which elutes at 270 ml from the column under the same conditions.				(4x1.5)

b.	Write a detailed note on hydrophobic interaction chromatography.					(9)

24.  	Explain the rate of drying as a function of free moisture contents for a hygroscopic material.																					(3x5)

(OR)

25.  	Write short notes on any three:

a.	Crystallization.

b.	Equipments of crystallization.

c.	Types of various driers and their function.

d.	Batch freeze drier.															(3x5)