Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CHEMICAL THERMODYNAMICS AND BIO THERMODYNAMICS															       					      Time: 3 hours

Subject Code: 09BT229			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Enthalpy (H) is defined as ___________.

2. 	Entropy is a measure of ___________ of a system.

3. 	Fugacity and pressure are numerically equal when the gas is in ____________.

4. 	Van Laar equation deals with activity coefficient in ___________.

5. 	Solid and liquid phase of a substance are in equilibrium at __________ and ___________.

6. 	At triple point, vapor pressure of solid as compared to that of liquid will be _________.

7. 	Domestic refrigerator usually works on ____________ cycle.

8. 	One ton of refrigeration capacity is equivalent to __________ BTU/min.

9. 	The number of available electrons per oxygen molecule is ___________.

10. 	The important substrate used in aerobic fermentation is ___________.

PART – B (5 x 3 = 15 MARKS)

11.	Give the fundamental property relation of thermodynamics.

12.	List out the significance of chemical potential.

13.	Define single reaction and multiple reactions.

14.	What is liquefaction? How is it achieved?

15.	What is the role of degree of reduction in stoichiometric calculations?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Show that for a gas obeying van der Waals equation of state,

CP – CV = R / (1 – 2a (V – b)2 / (RTV3 ) )

Where a and b are van der Waals constants.                				    				(8)

b.	Mercury has a density of 13.69 x 103 kg/m3 in the liquid state and 14.193 x 103 kg/m3 in the solid state, both measured at the melting point of 234.33 K at 1 bar. If the heat of fusion of mercury is 9.7876 kJ/kg what is the melting point of mercury at 10 bar? 	(7)

(OR)

17.	a.	Prove that (∂u / ∂T)V = Cv and (∂H / ∂T)P = Cp; for an ideal gas (∂u / ∂v)T = 0 and

(∂H / ∂p)T = 0.        		 	      		 				  						(10)

b.	Derive the expression for Entropy – Hat capacity relationship.		   				(5)

18.	a.	Discuss the Gibbs-Duhem equation and its various forms.             		  			(10)

b.	The fugacity of component 1 in binary liquid mixture of components 1 and 2 at 298 K and 20 bar is given by

f1 = 50 x1 – 80 x12 + 40 x13

where f1 is in bar and x1 is the mole fraction of component 1. Determine 	i. 	Fugacity f1 of pure component 1. 	ii.		Fugacity coefficient. 			   				(5)

(OR)

[P.T.O]

19. 	a.	At 300 K and 1 bar, the volumetric data for a liquid mixture of benzene and cyclohexane

are represented by V = 109.4 X 10-6 – 16.8 X 10-6 x – 2.64 X 10-6 x2, where ‘x’ is the mole fraction of benzene and V has the units of m3/mol. Find expressions for the partial molar volumes of benzene and cyclohexane.             		   		  	   				(7)

b.	How will you predict the effect of temperature and pressure on chemical potential? 				Explain.																		(8)

20. 	a.	Deduce the Clapeyron equation using the criterion dG = 0 (at constant T and P).		(6)	       b.	Prove that if Raoult’s law is valid for one constituent of a binary solution over the whole 	concentration range; it must also apply to the other constituent.		   			(9)

(OR)

21. 	a.	How will you predict the effect of temperature on an equilibrium constant? Explain.  	(7)

b.	The standard heat of formation and standard free energy of formation of ammonia at 298 K are -46,100 J/mol and -16,500 J/mol respectively. Calculate the equilibrium constant for the reaction.

N2 (g) + 3H2 (g)    →    2NH3 (g)

at 500 K assuming that the standard heat of reaction is constant in the temperature range 298 to 500 K.              							   		   						(8)

22.  a.	A mass of air is initially at 260°C and 700 KPa and occupies 0.028 m3. The air is expanded at constant pressure to 0.084 m3. A polytrophic process with n = 1.5 is then carried out, followed by a constant temperature process which completes a cycle. All the processes are reversible. Calculate 		i.	the amount of heat received and heat rejected in the cycle ii.	the efficiency of the cycle. 												(12)

b.	What is Joule Thompson Coefficient? If a gas cools on throttling, will it be positive or negative?								 		   							(3)

(OR)

23. 	a.	A vapor-compression cycle using NH3 as refrigerant is employed in an ice manufacturing 	plant. Cooling H2O at 288 K enter the condenser at a rate of 0.25 kg/sec and leaves at 300 	K. Ammonic at 294 K condenses at a rate of 0.50 kg/min. Enthalpy of liquid ammonia at 	294 K is 281.5 kJ/kg. The compressor efficiency is 90%. Saturated NH3 vapor at 258 K 	and enthalpy of 1426 kJ/kg enters the compressor. What is the power requirement of the 	compressor and refrigeration capacity in tons?       				   				(9)

b.	Write a note on heat pumps.      	  					   						(6)

24.	a.	Discuss the significance of thermodynamics of microbial growth.  		   			(7)

b.	What is maintenance coefficient? Explain its role in energetic analysis of microbial growth kinetics.					 			               								(8)

(OR)

25. 	Production of single-cell protein from hexadecane is described by the following reaction

C16H34 + a O2 + b NH3  →  c CH1.66O0.27N0.20 + d CO2 + e H2O.

Where CH1.66O0.27N0.20 represents the biomass. If RQ = 0.43, determine the stoichiometric coefficients.