Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF FLUIDS								       	       Time: 3 hours

Subject Code: 09CE202			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	State Newton’s law of viscosity.

2.	State Pascal’s law.

3.	Define: Stream function.

4.	State the Continuity equation.

5.	What is a Venturimeter?

6.	Define: Reynold’s number.

7.	Define: Hydraulic gradient line.

8.	What is meant by an Equivalent pipe?

9.	What do you mean by fundamental units?

10.	What do you mean by repeating variables?

PART – B (5 x 3 = 15 MARKS)

11.	What do you understand by Total pressure and Centre of pressure?

12.	Differentiate between free and forced vortex flow.

13.	Describe the Reynold’s experiment to demonstrate the two types of flow.

14.	What do you understand by the terms: major and minor energy losses in pipes?

15.	State the Buckingham’s π theorem. Why is this considered superior over the Rayleigh’s method for dimensional analysis?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	An oil of viscosity 5 Poise is used for lubrication between a shaft and sleeve. The diameter 	of the shaft is 0.5 m and it rotates at 200 rpm. Calculate the power lost in oil for a sleeve 	length of 100 mm. The thickness of oil film is 1mm.								(10)

b.	A hydraulic press has a ram of 20 cm diameter and a plunger of 3 cm diameter. It is used 	for 	lifting a weight of 30 kN. Find the force required at the plunger.				(5)

(OR)

17.	a.	Determine the total pressure on a circular plate of diameter 1.5 m which is placed vertically 		in water in such a way that the centre of the plate is 3 m below the free surface of water. 			Also find the position of centre of pressure.										(8)

b.	Find the magnitude and direction of the resultant force due to water acting on a roller gate of cylindrical form of 4 m diameter when the gate is placed on the dam in such a way that water is just going to spill. Take the length of gate as 8 m.							(7)

18.	If for a two dimensional potential flow, the velocity potential is given by φ = x(2y-1) determine the velocity at the point P(4,5). Also determine the value of the stream function Ψ at the point P.

(OR)

[P.T.O]

19.	State Bernoulli’s equation for flow of an incompressible fluid. Derive an expression for 	Bernoulli’s theorem from first principles and state the assumptions made for such a derivation.

20.	a.	Derive an expression for the velocity distribution for viscous flow through a circular pipe.																						(8)

b.	A laminar flow is taking place in a pipe of diameter 200 mm. The maximum velocity is       		1.5 m/s. Find the mean velocity and the radius at which this occurs.					(7)

(OR)

21.	a.	How would you distinguish between hydro dynamically smooth and rough boundaries?																						(5)

b.	A smooth pipe of diameter 400 mm and length 800 m carries water at the rate of 0.04 m3/s. Determine the head lost due to friction and wall shear stress. Take the kinematic viscosity of water as 0.018 Stokes.														(10)

22.	a.	What do you understand by Pipes in series and Pipes in parallel?					(5)

b.	An oil of specific gravity 0.9 and viscosity 0.06 Poise is flowing through a pipe of 	diameter 200 mm at the rate of 60 liters per second. Find the head lost due to friction for a 	500 m length of pipe. Also find the power required to maintain this flow.			(10)

(OR)

23.	A pipe of length 200 m is used for power transmission.  If 110 kW power is to be transmitted through the pipe in which water having a pressure of 490 N/cm2 at inlet is flowing, find the diameter of the pipe and efficiency of transmission, if the power drop over the length of pipe is 98 N/cm2. Take f = 0.0065.

24.	Using Buckingham’s π theorem, show that the velocity through a circular orifice is given by 
V = √2gH  φ (D/H, μ/ρVH) where H is the head causing flow, D is the diameter of orifice,  μ is coefficient of viscosity and ρ is mass density.

(OR)

25.	a.	What is the significance of the following dimensionless numbers: Reynold’s number, 	Froude number and Mach number?												(9)

b.	Write a detailed note on classification of hydraulic models.						(6)