Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF DEFORMABLE BODIES – I			      	       Time: 3 hours

Subject Code: 09CE205			       				               	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Hooke's law.

2.	What is the volumetric strain of a cube of side 'a'?

3.	What is the value of shear stress on principal planes?

4.	What are the stresses induced in the walls of thin shells?

5.	What is the maximum bending moment in a beam of span ‘l’ carrying concentrated load 'W' at mid-span?

6.	What is the bending moment at the free end of the cantilever of span 'l' carrying concentrated load at the free end?

7.	What is the bending stress in the neutral axis of a beam?

8.	What are the uses of leaf springs?

9.	Define polar modulus.

10.	Define stiffness of a spring.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the stress-strain diagram for mild steel.

12.	Write down the expression for volumetric strain in a thin spherical shell.

13.	Draw the B.M.D for a cantilever of span 'l' carrying uniformly a distributed load of ‘W’ kN/m over the entire span.

14.	What are the assumptions in the theory of simple bending?

15.	Write down the Torsion equation.

PART – C  (5 x 15 = 75 MARKS)

16.	The following data relate to a bar subjected to a tensile test:

Diameter of the bar =30mm

Tensile load =54kN

Gauge length =300mm

Extension of the bar =0.112mm

Change in diameter =0.00366mm

Calculate:	a.	Poisson's ratio 	b.	the value of three modulii.

(OR)

17.	An object of 100 N weight falls, by gravity, a vertical distance of 5m, when it is suddenly stopped by a collar at the end of a vertical rod of length 10m and diameter 20mm. The top of the body is rigidly fixed. Calculate the maximum stress and strain induced in the bar due to the impact. Take E=200GN/m2 for the material of the rod.

18.	At a point in a stressed body the principal stresses are 100MN/m2 (tensile) and 60MN/m2 (compressive). Determine the normal stress and the shear stress on a plane inclined at 500 to the axis of the major principal stress. Also calculate the maximum shear stress at the point.

(OR)

[P.T.O]

19.	A cylindrical shell 90cm long and with 20cm internal diameter having thickness of metal as 8mm is filled with a fluid at atmospheric pressure. If an additional 20cm3 of fluid is pumped into the cylinder, determine.

a.	Pressure exerted by the fluid in the cylinder

b.	The hoop stress induced.

20.	A beam AB 10m long carries a UDL of 20kN/m over its entire length together with a concentrated load of 50kN at the left end A and 80kN at the end B. The beam is to be supported at two props at the same level 6 m apart so that the reaction is same at each. Determine the position of the supports and draw the S.F and B.M. diagrams. Find the value of maximum B.M. Locate the point of contraflexure, if any.

(OR)

21.	A cantilever of span 5m carries concentrated loads of 1kN at E and A 1m and 5m from the fixed end. It also carries a uniformly distributed load of 2kN/m over DC of span 2m, such that DA=3m, CA=1m. Draw the S.F.D and B.M.D.

22.	Determine the dimensions of a joist of timber for span 8 m to carry a brick wall 200mm thick and 5m high, if the density of the brick work is 1850kg/m3 and the maximum permissible stress is limited to 7.5 MN/m2.  The depth of the joist is twice the width.

(OR)

23.	A laminated steel spring 1m long is to support a central load of 5.8kN. If the maximum 	deflection of the spring is not to exceed 45mm and maximum stress should not exceed 	300MN/m2,

Calculate:

a.	The thickness of the leaf.

b.	Their number if each plate is to be 80mm wide. Assume E=200GN/m2.

24.	A solid circular shaft transmits 75kW power at 200rpm. Calculate the shaft diameter, if the twist in the shaft is not to exceed 10 in 2m length of shaft, and shear stress is limited to 50MN/m2. Take C=100GN/m2.

(OR)

25.	A close coiled helical spring is to have a stiffness of 900N/m in compression, with a maximum load of 45N and a maximum shearing stress of 120N/mm2. The solid length of the spring (i.e., coils touching) is 45mm.Determine:

a.	The wire diameter.				b.	The mean coil radius.

c.	The number of coils.
Assume the modulus of rigidity of material of spring, C=0.4x105 N/mm2.