Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF DEFORMABLE BODIES II			      	       Time: 3 hours

Subject Code: 09CE206			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the maximum deflection in a simply supported beam of span ‘l’ carrying a concentrated load ‘W’ at mid span?

2.	What is the load on a conjugate beam?

3.	Define Slenderness ratio.

4.	Distinguish between long column and short column.

5.	State Maximum Principal Stress Theory.

6.	Distinguish between thin and thick cylinder.

7.	Define shear centre.

8.	What is the stress in any layer of a curved bar of radius R and area A, subjected to a bending moment (M), tending to increase the curvature?

9.	What is unsymmetrical bending?

10.	Write down the expression for deflection of a beam due to unsymmetrical bending.

PART – B  (5 x 3 = 15 MARKS)

11.	State Mohr’s theorem.

12.	What are the assumptions in Euler’s theory?

13.	State the assumptions in Lame’s theory.

14.	What are the assumptions in Winkler – Bach theory?

15.	What are the causes of unsymmetrical bending?

PART – C  (5 x 15 = 75 MARKS)

16.	A cantilever 150 mm wide and 200 mm deep projects 2 m out of a wall and carries a concentrated load of 40 KN at the free end. Determine the slope and deflection of the cantilever at the free end. Young’s modulus E = 2.1x105 N/mm2.

(OR)

17.	A cantilever 2 m long carries a concentrated load of 15 KN at 1m from the fixed end and a load of 10 KN at the free end. Determine the deflection at the free end. Use conjugate beam method.

Assume E= 2x105 N/mm2, I=15x106 mm4.

18.	Derive Euler’s formula when one end of the column is fixed and the other is free.

(OR)

19.	A hollow cast iron column is 4.5 m long and fixed at both ends. The internal diameter and external diameter of the column are 160 mm and 200 mm respectively. Determine the safe load by Rankine’s formula using a factor of safety of 4. бc = 550 MN/mm2 ;  α=1/1600.

20.	A pipe of 200 mm internal diameter and 50 mm thickness carries fluid at a pressure of 10 MPa. Determine the maximum and minimum circumference stresses across the section. Also sketch the radial stress (pressure) distribution and circumferential stress distribution across the section.

(OR)

[P.T.O]

21.	A shaft is subjected to a maximum torque of 10 KNm and a maximum bending moment of

7. 5 KNm at a particular section. If the allowable equivalent stress in simple tension is 160 MPa, determine the diameter of the shaft using Maximum Shear Stress theory.

22.	A channel has flanges 120 mm x 20mm and web 160 mm x10 mm. Determine the shear centre of the channel.

(OR)

23.	A steel ring has a rectangular cross - section 75 mm in the radial direction and 45 mm perpendicular to the radial direction. The mean radius of the ring is 150 mm and maximum tensile stress in limited to 180 MPa. Determine the tensile load the ring can carry.

24.	An 80 mm x 80 mm angles section is used as a freely supported beam with one leg vertical. On the application of bending moment in the vertical plane YY, the mid section of the beam deflects in a clockwise direction at 300 15’ to the vertical.   X=Y = 23.4 mm and Ixx = Iyy= 87.36x10-8 m4.  Determine:

a.	The second moment of area of the section about its principal axis.

b.	The bending stress at the left corner of the section if the bending moment is 1.5 kN-m.

(OR)

25.	A beam of T-section (Flange: 100 mm x 20 mm, Web: 150 mm x 10 mm) is 2.5 m in length and simply supported at the ends. It carries a load of 3.2 KN inclined at 200 to the vertical and passing through the centroid of the section. Young’s modulus E=2x105 N/mm2.

Determine:		a.	Maximum tensile stress. 		b.	Maximum compressive stress.