Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: STRUCTURAL ANALYSIS –I							       	       Time: 3 hours

Subject Code: 09CE208		       						                    	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.  The equation to find the degree of static indeterminacy of a pin jointed plane frame is 	___________________. analysis

2.	The strain energy stored by a member so deformed is equal to the _______________.

3.	For a simply supported beam, the influence line diagram for Va is _______________.

4.	Draw a diagram to show two wheel loads 80 kN and 200 kN spaced 2m apart move on a 20m  span with the leading load at 4m.

5. 	Define horizontal thrust.

6. 	The horizontal distance between the lower hinges is called ____________.

7. 	The tension in the cable is found by _____________.

8. 	Name the methods used for the analysis of Pin-jointed space frames ________________.

9. 	Define a built-in beam.

10. 	What are fixed end moments?

PART – B (5 x 3 = 15 MARKS)

11. 	Define the Principle of Superposition.

12. 	What is meant by Influence line diagram?

13.	A three hinged arch of span l and rise h carries a uniformly distributed load of w per unit run over the entire span. Write the expression for horizontal thrust.

14.	Draw a three- hinged stiffening girder suspension bridge.

15. 	Explain Clapetron’s theorem of three moments.

PART – C (5 x 15 = 75 MARKS)

16. 	Using Strain energy and Castigilano’s theorem find the vertical deflection of the joint D of the frame shown in the figure. All the members have the same cross sectional area A. The diagonal members are at 45° with the horizontal.

A                             B                                 C

F	                             E                                      D

P

(OR)

[P.T.O]

17. 	Using Maxwell’s method, find the vertical and horizontal deflection of the joint C of the pin jointed truss shown in the figure. The area of the horizontal member is 150mm2 and the area of the members AC and BC are 200mm2 each. Take E=200 kN/mm2.

C

4.5m

A        θ                                    B

6m               6m

18. 	Two point loads of 100 kN and 200 kN spaced 3m apart cross a girder of span 15m from left to right with the 100 kN load leading. Draw the influence line for shear force and bending moment and find the value of maximum shear force and bending moment at a section D, 6m from the left hand support. Also find the absolute maximum bending moment due to the given load systems.

(OR)

19. 	A train of 5 wheel loads crosses a simply supported beam of span 22.5m. Using influence lines calculate the maximum positive and negative shear force at mid span and absolute maximum bending moment anywhere in the span.

120kN       160kN     400kN    260kN    240kN

2.5m         2.5m         2.5m       2.5m

20. 	A three hinged arch of span 16m has its abutments A and B at a depth of 4m and 8m below the crown C and 6m and 10m towards the LHS and RHS respectively. The arch carries a uniformly distribution load of 30 kN/m throughout its span and also two concentrated load of 100 kN and 200 kN at a distance of 7m and 3m from the right hand support. Determine the horizontal thrust and vertical reactions at the hinged supports A and B.

(OR)

21. 	A three-hinged parabolic arch has span of 20m and a rise of 5m. It carries a uniformly distributed load of 20 kN per meter over the left hand of the span and a point load of 120 kN at 5m from the right end. Find the bending moment, normal thrust and radial shear at section 4m from the left end.

22. 	A cord supported at its ends 40m apart carries load 20 kN, 10 kN and 12 kN at distance 10m, 20m and 30m from the left end. If the point on the cord where the 10 kN load is supported is 13m below the level of end supports, determine:		 a.	the reaction at the supports

b.	the tension in the different parts of the cord 		 c.	the total length of the cord.

(OR)

23. 	The three-hinged stiffening girder of a suspension bridge of span 120m is subjected to two point load 240 kN and 300 kN at distance 25m and 80m from the left end. Find the shear force and bending moment for the girder at a distance of 40m from the left end. The supporting cable has a depth of 8m. Find also the maximum tension in the cable, and draw the B.M diagram for the girder.

[P.T.O]

24. 	A fixed beam of 6m span supports two point loads of 300 kN each at 2 meters from each end. Find the fixing moments at the ends and draw the B.M and S.F diagrams. Find also the Central deflection. Take I=9x108mm4 and E=200kN/mm2.

(OR)

25. 	A continuous beam consists of three successive spans of 8m, 10m and 6m and carries loads of 60 kN/m, 40kN/m and 80kN/m respectively on the spans. Determine the bending moment and reaction at the supports. Apply theorem of three moments.