Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: HIGHWAYS AND RAILWAYS ENGINEERING		      	       Time: 3 hours

Subject Code: 09CE209				       				        	 	       Maximum Marks: 100

#### Answer ALL questions 
Use of IRC:37-2001 and IRC:58-2002 is permitted

PART – A  (10 x 1 = 10 MARKS)

1. 	How are urban roads classified?

2.	What is a ‘chamber’? State its necessity.

3.	What is meant by ‘lag distance’?

4.	How is terrain classified?

5.	Write down the Westergaard’s stress equation when the wheel load is at the edge region.

6.	Draw the typical cross-section showing the various layers of flexible pavements.

7.	State the different gauges adopted in Indian Railways.

8.	What is meant by creep in rail?

9.	What is a turnout?

10.	Where will the catch sidings be provided?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between ‘right of way’ and ‘carriage way’.

12.	State the need for widening the pavements at horizontal curves. Also mention their components.

13.	List the factors to be considered for pavement design.

14.	Define ‘sleeper density’.

15.	State the requirements of railway stations.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the factors controlling the highway alignments.

(OR)

17.	a.	Enumerate the importance of highway transportation.								(10)

b.	Brief the objectives of CRRI.													(5)

18. 	a.	Describe the factors affecting the stopping sight distances.							(6)

b.	The radius of a horizontal curve is 240 m. The design speed is 80 kmph. Assuming the    	co-efficient of friction as 0.15, calculate:

i. 	The super elevation if full lateral friction is assumed to develop.

ii. 	The equilibrium super elevation if pressure on inner and outer wheels should be equal.

iii. 	The co-efficient of friction needed if no super elevation is provided.			(9)

(OR)

[P.T.O]

19.	a. 	Discuss the factors to be considered while designing the length of summit curves.	(5)

b. 	A valley curve is formed by a descending grade of 1 in 25 meeting an ascending grade of 1 		in 30. Design the length of valley curve to fulfill both comfort condition and head light			sight distance requirements for a design speed of 80 kmph. Assume allowable rate of 			change of centrifugal acceleration as 0.6 m/sec3.									(10)

20.	Design a flexible pavement as per IRC: 37-2001 for a segment of a State Highway with the following data:

Present Traffic   		= 200 CVPD in both directions

No. of Lanes      		= 2

Construction Period   	= 1.5 years

CBR Test Results

| Penetration in mm | Load in kg |
| --- | --- |
| 0 | 0 |
| 0.5 | 5 |
| 1.0 | 16.2 |
| 1.5 | 28.1 |
| 2.0 | 40 |
| 2.5 | 48.5 |
| 3.0 | 56.5 |
| 4.0 | 67.5 |
| 5.0 | 75.2 |
| 7.5 | 89 |
| 10 | 99.5 |
| 12.5 | 106.5 |

(OR)

21.	Design a cement concrete pavement and its joint reinforcement for the following data:

Present traffic 					= 250 CVPD

Design Life 						= 20 years

Modulus of Sub-Grade Reaction 	= 20.8 kg/cm2/cm

Flexural Strength of Concrete 		= 45 kg/cm2

Slab size 						= 4.0 m x 4.0 m

Assume all other relevant data suitably.

22.	Bring out the features, advantages and disadvantages of different types of sleepers.

(OR)

23.	a.	What are the ideal characteristics of elastic fastenings?							(5)

b.	Draw a typical cross section of B.G. double track in cutting and mark its components and 	dimensions.																	(10)

24.	Calculate all the necessary elements required to set out a 1 in 8½ turnout taking off from a straight B.G track with its curve starting from the toe of the switch (i.e.) tangential to the gauge face of the outer main rail and pass through theoretical nose of crossing. The heel divergence (d) is 11.4 cm.

(OR)

25.	a.	What is interlocking of signals and points?				      	         				(3)

b.	Describe the working principle of mechanical method of interlocking. 	                   	(12)