Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: REINFORCED CONCRETE STRUCTURES – I			       Time: 3 hours

Subject Code: 09CE210			       						       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the minimum grade of concrete for RC as per IS456-2000?

2.	What is factored load?

3.	Define two way slab.

4.	Where are the lintel beam provided in the structure?

5.	Define column.

6.	What is the effective length of a column?

7.	Write the formula to calculate the effective width of flange of T beam.

8.	For continuous beams, frames  I0 may be assumed as __________ times the effective span.

9.	How will you calculate the depth of footing below the ground level?

10.	A long column has a slenderness ratio greater than __________.

PART – B (5 x 3 = 15 MARKS)

11.	A rectangular beam with b= 300 mm and d= 500 mm is reinforced with 4 – 20 mm diameter bars. If the shear force to be restricted is 75 kN, obtain the vertical web reinforcement required, if concrete grade be M15 and be of Fe 415 grade.

12. 	What are the different cases to be considered for the position of neutral axis for T beam design?

13. 	A 4 m high column is effectively held in position at both ends and restrained against the rotation at one end. Its diameter is restricted to 40 cm. Find the slenderness ratio and minimum eccentricity.

14.  Write down the calculations of neutral axis depth of a T beam of balanced section in Working 	stress method.

15. 	A reinforced rectangular concrete tied column and of sides 400 mm x500 mm is provided with 4-32 mm diameter bars. Calculate the permissible axial load on the column if the effective length be 4m. Use M20 mix and Fe 250 steel.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the design concept for working stress method and ultimate load method.

(OR)

17. 	A singly reinforced beam of 20cm wide and 40cm depth upto the centre of reinforcement. Find the reinforcement required if it has to resist a moment of 40KNm. Assume  M20 mix and Fe250 grade steel.

18. 	Design a two way slab for a room 5.5 m x 4 m clear in size, if the super imposed load is 5KN/m2.Use M20 mix and Fe250 grade steel. Take  edges  simply supported  with  corners not held down.

(OR)

19. 	Design a simply supported roof slab for a room 8m x 3.5m clear in size, if the superimposed load is 5KN/m2.  Assume M20 mix and Fe415 grade steel.

[P.T.O]

20.	A column  is required to carry a  load of 200,000 N inclusive of its own weight. The column is a  tied column and can be considered as a short one. Calculate dimensions of the column so as to obtain:

a.	The smallest square column.

b.	The most economical square column.

c.	A column of size 400 mm x 500 mm.

Use M20 mix and Fe250 grade steel.

(OR)

21. 	Design a rectangular isolated column footing of uniform thickness for a reinforced concrete column of size 400 mm x 600 mm and carrying an axial load of 2500 kN inclusive of its own weight safe bearing capacity of soil =150,000N/m2 . Use M20 mix and Fe415 grade steel.

22. 	A floor consists of 100 mm thick slabs with T beam at 2 m centers, the simply supported span of the beams being 6.45 m.Obtain the dimensions of the beam along with reinforcements. if the live load on the slab be 7000 N/m. Assume CBC = 5N/mm2, ST = 140N/mm2, k=0.405. Assume unit weight of concrete = 24000 N/m2. Use Fe415 steel.

(OR)

23. 	Write the procedure in design of T beams by working stress method.

24. 	Design a cantilever sunshade projecting 1 m beyond the face of a 250 mm thick wall of a residential building. Assume live load as 2000 N/m2. Use M20 mix and Fe250 grade steel.

(OR)

25. 	A square reinforced concrete column of sides 600 mm is reinforced with 8-40mm diameter bars longitudinally and 16 mm spiral binding with 50 mm pitch. The diameter of the core is 500 mm. Use M20 mix and Fe250 grade steel.

a.	Calculate the permissible axial load for a short column of the cross section

b.	What will be the permissible load when effective length is 9m?