Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: GEOTECHNICAL ENGINEERING						       Time: 3 hours

Subject Code: 09CE212				       				              	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Soil transported by gravitational force is called ___________.

2.	If void ratio of soil mass is 0.65, calculate its porosity.

3.	If uniformity coefficient is twice the coefficient of curvature, determine the ration between D60 and D30.

4.	The water content corresponding to the arbitrary limit between liquid state to plastic state is termed as ____________.

5.	If the velocity of flow is 0.0414 cm/sec and porosity of soil is 0.35, determine seepage velocity.

6.	A coarse grained soil has a void ratio of 0.78 and specific gravity 2.67. Calculate critical hydraulic gradient.

7.	If liquid limit of clay is 60%, determine compression index.

8.	Consolidation settlement can be computed by _________ and _________ methods.

9.	Shear test that can be calculated both in laboratory and field is _________.

10.	Define factor of safety against sliding.

PART – B (5 x 3 = 15 MARKS)

11.	Define Soil Engineering.

12.	Write the significance of toughness index.

13.	Give the importance of permeability.

14.	What is pre-consolidation pressure?

15.	Draw the Mohr circle at failure and strength envelope corresponding to unconfined compression test.

PART – C (5 x 15 = 75 MARKS)

16.	A wet soil sample of mass 1.9 kg had a volume of 945 cc. After oven drying its mass was reduced to 1.7 kg. The specific gravity of solids was found to be 2.7. Determine moisture content, bulk density, dry density, void ratio, porosity and degree of saturation.

(OR)

17.	Explain the procedure for calculating specific gravity and water content by pycnometer method.

18.	The following data are obtained from laboratory tests on two soil samples:

|  | Percentage Finer |  |
| --- | --- | --- |
| Particle Size, mm | Sample A | Sample B |
| 4.75 | 100 | 100 |
| 2.00 | 95 | 100 |
| 0.426 | 82 | 48 |
| 0.075 | 75 | 25 |
| Atterberg limits |  |  |
| Liquid limit, % | 48 | 28 |
| Plastic limit, % | 24 | 24 |
| Classify the Sample A & B based on BIS |  |  |

(OR)

19.	Write a detailed note on consistency limit and its importance.

[P.T.O]

20.	Calculate the coefficient of permeability of a soil sample 60mm in height and 5000 sq.mm in cross sectional area, if a quantity of water equal to 450 cc passed down in 10 minutes under an effective constant head of 400 mm. On oven drying the test specimen had a mass of 495 g. Taking the specific gravity of solids as 2.65, calculate the seepage velocity of water during the test.

(OR)

21.	What is flow net? State the applications of flow net. How can seepage through anisotropic soil be determined?

22.	Explain spring analogy method for consolidation.

(OR)

23.	A saturated soil has a compression index 0.25, void ratio at stress 10 kN / sq.m is 2.02 and coefficient of permeability 3.4 x 10-7 mm / s. Compute change in void ratio if increase in stress is 19 kN/ sq.m. Also find volume of compressibility, Coefficient of compressibility and settlement of layer is 5 m thick.

24.	The results of triaxial test conducted on three samples of a soil are given below. Obtain the shear strength parameters of the soil.

| Test No. | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Cell pressure, kPa | 100 | 200 | 400 |
| Deviator Stress at failure, kPa | 375 | 575 | 973 |

(OR)

25.	Explain friction circle method of analysis for stability of slopes.