Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF MASONRY AND TIMBER STRUCTURES	       Time: 3 hours

Subject Code: 09CE213			       						       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the basic requirements of a good structure?

2.	Mention the lateral loads that act on a structure.

3.	How can one determine the allowable compressive strength of brick masonry?

4.	What are the stresses that act on an eccentrically loaded masonry wall?

5.	Define Kern area.

6.	When should one use the bed block?

7.	What is the type of load, a shear wall will take?

8.	The flexural member designed to carry the load from brick masonry above an opening is termed as _________ .

9.	What is meant by a flitched beam?

10.	What is a built-up column?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between the working stress method and limit state method.

12.	Give the classification of masonry walls.

13.	State the conditions for stability of masonry structures on soil.

14.	Explain the behavior of reinforced masonry walls.

15.	Brief a note on the factors affecting the strength of timber.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the choice of different construction materials for different applications.

b.	Discuss the classification of structures.

(OR)

17.	Explain the various structural loads to be accounted in structural design.

18.	Determine the allowable axial load on a brick pillar 300mm x 500mm with crushing strength of

bricks 7.5 MPa using CM 1 : 5, size of brick being 200 x 100 x 75mm. Effective height of pillar is 4.5m.

(OR)

19.	Design an interior cross wall for a two storeyed building to carry 100 mm thick R.C.C. slabs

with 3m ceiling height. The wall is unstiffened and supports a 2.65m wide slab.  Live load on roof is 1.5 kN/m2.  Live load on floor is 2 kN/m2; Weight of the 80 mm thick terrace is 1.96 kN/m2; Weight of floor finish is 0.8 kN/m2.

20.	A masonry wall 350mm thick of height 3m carries a load of 120 kN/m length. Design a reinforced concrete footing for this wall if bearing capacity of the soil over which it rests is 110 kN/m2 . Density of soil is 16kN/m3 with the angle of repose of 320.

(OR)

[P.T.O]

21.	A masonry dam of trapezoidal section has its top width equal to 4m, height 25m, water face vertical and water level with top. If the masonry weighs 22.6 kN/ m3, find the minimum width of the base to avoid tensile stresses, and determine the maximum compressive stress for this base width. If the coefficient of friction between base and soil is 0.6, check the stability of the dam.

22.	Design a wall of 4.5m length and 3m height to resist a horizontal earthquake force in its plane. Assume the seismic load to be uniformly distributed across the height of the wall. Earthquake acceleration is 0.1 g, the wall is tied with metal anchors at the top and bottom supports.

(OR)

23.	Design a free standing wall of height 1.6m with design seismic coefficient 0.12 for:

a.	No tension condition.

b.	0.1N/mm2 tensile strength.

24.	A flitched beam consists of two teak wood joists of 15cm x 30cm strengthened by a steel plate 12mm thick and 30cm deep one on either side of the joist. If the maximum stress in the wooden joist is 7N/mm2, find the maximum stress attained in steel.  Find also the moment of resistance of the section. Take Es = 20Ec

(OR)

25.	A spaced column is 4.0 m long carrying an axial compressive force of 100 kN.  Design the column using teak wood. Assume grade I timber and inside location.