Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: APPLIED HYDRAULICS AND FLUID MACHINES  		       Time: 3 hours

Subject Code: 09CE216			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Differentiate between uniform flow and non-uniform flow.

2.	Write down the equation for a gradually varied flow.

3.	Name the types of drags.

4.	List three characteristics of boundary layer.

5.	If the area of a jet is 0.00441 m2, velocity of the jet is 25 m/s and mass density of water is 1000 kg/m3, obtain the force on a vertical stationary plate.

6.	State few applications of impulse-momentum equation.

7.	Differentiate Reaction and Impulse turbines.

8.	What is the use of breaking jet?

9.	What do you mean by priming of a pump?

10.	What is meant by negative slip and when will it occur?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by hydraulically best section?

12.	Explain any two methods used to prevent boundary flow separation.

13.	A jet of water, 75mm in diameter, issues with a velocity of 30 m/s and impinges on a moving plate with a velocity of 8 m/s. Find the force exerted by the jet on the plate and the work done.

14.	Explain the different types of efficiencies in Turbines.

15.	What is meant by cavitation and what are its effects?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A rectangular channel is to be dug in the rocky portion of a soil. Find its most economical 	c/s, if it is to convey 12 m3/s water with an average velocity of 3m/s. Take Chezy’s 	constant C = 50.	    						    									(8)

b.	Explain the specific energy curve diagram. 			    							(7)

(OR)

17.	a.	Derive the expression for loss of head in a hydraulic jump.						(5)

b.	In a rectangular channel of 0.5 m width, a hydraulic jump occurs at a point where depth of water flow is 0.15 m and Froude number is 2.5. Determine:

i.	Specific Energy		ii.	Critical and subsequent depths   	iii. 	Head loss

iv.	Energy dissipated 														(10)

18.	The velocity distribution in the boundary layer is given by u/U=(y/)1/7. Calculate the 	following:

a.	Displacement thickness		b.	Momentum thickness		c.	Shape factor

d.	Energy thickness

(OR)

19.	a. 	Explain the following terms: Streamlined bodies, Stagnation point and Magnus Effect.

(6)

[P.T.O]

b.  	A kite weighing 9.8 N and having an area 1m2 makes an angle of 7o to horizontal when flying in a wind of 36 km/h. If the pull on the string attached to the kite is 49 N and it is inclined to the horizontal at 45o, calculate the lift and drag coefficients.  			(9)

20.	A jet of water having a velocity of 40 m/s strikes a curved vane that is moving with a velocity of 20 m/s. The jet makes an angle of 30o with the direction of motion of vane at inlet and leaves at an angle of 90o to the direction of motion of vane at outlet. Draw the velocity triangles at inlet and outlet and determine the vane angles at inlet and outlet so that the water enters and leaves the vanes without shock.

(OR)

21.	300 liters per second of water is flowing in a pipe. The pipe is bent by 120°. The pipe bend measures 360 mm × 240 mm and volume of the bend is 0.14 m3. The pressure at the entrance is 72 kN/m2 and the exit is 2.4 m above the entrance section. Find the force exerted on the bend.

22.	A single jet pelton wheel runs at 300 r.p.m under a head of 510 m. The jet diameter is 200 mm, its deflection inside the bucket is 165o and its relative velocity is reduced by 15% due to friction. Determine:

a.	water power			b.	resultant force on the bucket		c.	 overall efficiency.

Take coefficient of velocity = 0.98 and speed ratio = 0.46.

(OR)

23.	a.	Write short notes on draft tube.						                					(3)

b.	An inward flow reaction turbine has an external diameter of 1m and its breadth at inlet is 250 mm. If the velocity of water passing the inlet is 2 m/s, find the weight of water passing through the turbine per second. Assume 10% of the area of flow is blocked by blade thickness. If the speed of the runner is 210 rpm and the guide blade makes an angle of 10º to the wheel tangent, draw the inlet velocity triangle and find:

i.	Runner vane angle at inlet.						ii.	Velocity of wheel at inlet.

iii. 	Absolute velocity of water leaving guide vanes.	iv. 	Relative velocity of water 	entering the runner blade.         												(12)

24.	A centrifugal pump, in which water enters radially, delivers water to a height of 165 mm. The impeller has a diameter of 360 mm, width 180 mm at inlet and the corresponding dimensions at the outlet are 720 mm, and 90 mm respectively; its rotational speed is 1200 rpm. The blades are curved backward at 30° to the tangent at exit and the discharge is 0.389 m3/s. Determine:

a.	Theoretical head		b.	Manometric efficiency

c.	Pressure rise across the impeller assuming losses equal to 12 percent of velocity head at 	exit.

d.	Vane angle at inlet 	e.	Power required to drive the pump.

(OR)

25.	Describe the principle and working of a reciprocating pump with neat sketches showing its components and fitted with air vessels.